5 Nov. 2014

c. COMMON CODE TABLES TO BINARY AND ALPHANUMERIC CODES

COMMON CODE TABLE C–1:	Identification of originating/generating centre

F1F2 for alphanumeric codes

F3F3F3 for alphanumeric codes

Code table 0 in GRIB Edition 1/Code table 0 01 033 for BUFR Edition 3

Octet 5 in Section 1 of GRIB Edition 1/Octet 6 in Section 1 of BUFR Edition 3

COMMON CODE TABLE C–2:	Radiosonde/sounding system used

Code table 3685 – rara (Radiosonde/sounding system used) – for alphanumeric codes

Code table 0 02 011 (Radiosonde type) in BUFR

COMMON CODE TABLE C–3:	Instrument make and type for water temperature profile measurement
with fall rate equation coefficients

Code table 1770 – IXIXIX (Instrument type for XBT, with fall rate equation coefficients) – for
	alphanumeric codes

Code table 0 22 067 (Instrument type for water temperature/salinity profile measurement) in BUFR

COMMON CODE TABLE C–4:	Water temperature profile recorder types

Code table 4770 – XRXR (Recorder type) – for alphanumeric codes

Code table 0 22 068 (Water temperature profile recorder types) in BUFR

COMMON CODE TABLE C–5:	Satellite identifier

I6I6I6 for alphanumeric codes

Code table 0 01 007 in BUFR

Code used in GRIB Edition 2

COMMON CODE TABLE C–6:	List of units for TDCFs

(Used only in Volume I.2, Parts B and C)

COMMON CODE TABLE C–7:	Tracking technique/status of system used

Code table 3872 – sasa for alphanumeric code

Code table 0 02 014 in BUFR

COMMON CODE TABLE C–8:	Satellite Instruments

Code table 0 02 019 in BUFR

COMMON CODE TABLE C–11:	Originating/generating centres

BUFR 0 01 035

CREX Edition 2, ooooo in Group Poooooppp in Section 1

GRIB Edition 2, Octets 6–7 in Section 1

BUFR Edition 4, Octets 5–6 in Section 1

COMMON CODE TABLE C–12:	Sub-centres of originating centres defined by entries in Common Code
tables C–1 or C–11

BUFR 0 01 034

BUFR Edition 3, Octet 5 in Section 1

BUFR Edition 4, Octets 7–8 in Section 1

GRIB Edition 1, Octet 26 in Section 1

GRIB Edition 2, Octets 8–9 in Section 1

CREX Edition 2, ppp in Group Poooooppp in Section 1

COMMON CODE TABLE C–13:	Data sub-categories of categories defined by entries in BUFR Table A

BUFR Edition 4, Octet 12 in Section 1 (if = 255, it means other sub-category or undefined)

CREX Edition 2, mmm in Group Annnmmm of Section 1

COMMON CODE TABLE C–14:	Atmospheric chemical or physical constituent type

Code Table 4.230 in GRIB 2

COMMON CODE TABLE C–1:	Identification of originating/generating centre

F1F2 for alphanumeric codes

Common Code table	F3F3F3 for alphanumeric codes

Code table 0 in GRIB Edition 1/Code table 0 01 033 in BUFR Edition 3

Octet 5 in Section 1 of GRIB Edition 1/Octet 6 in Section 1 of BUFR Edition 3

Octet 5 in Section 1

Code figure	Code figure	of GRIB Edition 1

for F1F2	for F3F3F3	Octet 6 in Section 1

of BUFR Edition 3

00	000	0	WMO Secretariat

01–09: WMCs

01	001	1	Melbourne

02	002	2	Melbourne

03	003	3	)

04	004	4	Moscow

05	005	5	Moscow

06	006	6	)

07	007	7	US National Weather Service – National Centres

for Environmental Prediction (NCEP)

08	008	8	US National Weather Service Telecommunications Gateway
				(NWSTG)

09	009	9	US National Weather Service – Other

10–25: Centres in Region I

10	010	10	Cairo (RSMC)

11	011	11	)

12	012	12	Dakar (RSMC)

13	013	13	)

14	014	14	Nairobi (RSMC)

15	015	15	)

16	016	16	Casablanca (RSMC)

17	017	17	Tunis (RSMC)

18	018	18	Tunis – Casablanca (RSMC)

19	019	19	)

20	020	20	Las Palmas

21	021	21	Algiers (RSMC)

22	022	22	ACMAD

23	023	23	Mozambique (NMC)

24	024	24	Pretoria (RSMC)

25	025	25	La Réunion (RSMC)

26–40: Centres in Region II

26	026	26	Khabarovsk (RSMC)

27	027	27	)

28	028	28	New Delhi (RSMC)

29	029	29	)

30	030	30	Novosibirsk (RSMC)

31	031	31	)

32	032	32	Tashkent (RSMC)

33	033	33	Jeddah (RSMC)

34	034	34	Tokyo (RSMC), Japan Meteorological Agency

Octet 5 in Section 1

Code figure	Code figure	of GRIB Edition 1

for F1F2	for F3F3F3	Octet 6 in Section 1

of BUFR Edition 3

35	035	35	)

36	036	36	Bangkok

37	037	37	Ulaanbaatar

38	038	38	Beijing (RSMC)

39	039	39	)

40	040	40	Seoul

41–50: Centres in Region III

41	041	41	Buenos Aires (RSMC)

42	042	42	)

43	043	43	Brasilia (RSMC)

44	044	44	)

45	045	45	Santiago

46	046	46	Brazilian Space Agency ­ INPE

47	047	47	Colombia (NMC)

48	048	48	Ecuador (NMC)

49	049	49	Peru (NMC)

50	050	50	Venezuela (Bolivarian Republic of) (NMC)

51–63: Centres in Region IV

51	051	51	Miami (RSMC)

52	052	52	Miami (RSMC), National Hurricane Centre

53	053	53	Montreal (RSMC)

54	054	54	)

55	055	55	San Francisco

56	056	56	ARINC Centre

57	057	57	US Air Force – Air Force Global Weather Central

58	058	58	Fleet Numerical Meteorology and Oceanography Center, 
				Monterey, CA, United States

59	059	59	The NOAA Forecast Systems Laboratory, Boulder, CO,
				United States

60	060	60	United States National Center for Atmospheric Research
				(NCAR)

61	061	61	Service ARGOS – Landover

62	062	62	US Naval Oceanographic Office

63	063	63	International Research Institute for Climate and Society (IRI)

64–73: Centres in Region V

64	064	64	Honolulu (RSMC)

65	065	65	Darwin (RSMC)

66	066	66	)

67	067	67	Melbourne (RSMC)

68	068	68	Reserved

69	069	69	Wellington (RSMC)

70	070	70	)

71	071	71	Nadi (RSMC)

72	072	72	Singapore

73	073	73	Malaysia (NMC)

Octet 5 in Section 1

Code figure	Code figure	of GRIB Edition 1

for F1F2	for F3F3F3	Octet 6 in Section 1

of BUFR Edition 3

74–99: Centres in Region VI

74	074	74	UK Meteorological Office ­ Exeter (RSMC)

75	075	75	)

76	076	76	Moscow (RSMC)

77	077	77	Reserved

78	078	78	Offenbach (RSMC)

79	079	79	)

80	080	80	Rome (RSMC)

81	081	81	)

82	082	82	Norrköping

83	083	83	)

84	084	84	Toulouse (RSMC)

85	085	85	Toulouse (RSMC)

86	086	86	Helsinki

87	087	87	Belgrade

88	088	88	Oslo

89	089	89	Prague

90	090	90	Episkopi

91	091	91	Ankara

92	092	92	Frankfurt/Main

93	093	93	London (WAFC)

94	094	94	Copenhagen

95	095	95	Rota

96	096	96	Athens

97	097	97	European Space Agency (ESA)

98	098	98	European Centre for Medium-Range Weather Forecasts

(ECMWF) (RSMC)

99	099	99	De Bilt

Additional Centres

Not applicable	100	100	Brazzaville

Not applicable	101	101	Abidjan

Not applicable	102	102	Libya (NMC)

Not applicable	103	103	Madagascar (NMC)

Not applicable	104	104	Mauritius (NMC)

Not applicable	105	105	Niger (NMC)

Not applicable	106	106	Seychelles (NMC)

Not applicable	107	107	Uganda (NMC)

Not applicable	108	108	United Republic of Tanzania (NMC)

Not applicable	109	109	Zimbabwe (NMC)

Not applicable	110	110	Hong-Kong, China

Not applicable	111	111	Afghanistan (NMC)

Not applicable	112	112	Bahrain (NMC)

Not applicable	113	113	Bangladesh (NMC)

Not applicable	114	114	Bhutan (NMC)

Not applicable	115	115	Cambodia (NMC)

Not applicable	116	116	Democratic People's Republic of Korea (NMC)

Octet 5 in Section 1

Code figure	Code figure	of GRIB Edition 1

for F1F2	for F3F3F3	Octet 6 in Section 1

of BUFR Edition 3

Not applicable	117	117	Islamic Republic of Iran (NMC)

Not applicable	118	118	Iraq (NMC)

Not applicable	119	119	Kazakhstan (NMC)

Not applicable	120	120	Kuwait (NMC)

Not applicable	121	121	Kyrgyzstan (NMC)

Not applicable	122	122	Lao People's Democratic Republic (NMC)

Not applicable	123	123	Macao, China

Not applicable	124	124	Maldives (NMC)

Not applicable	125	125	Myanmar (NMC)

Not applicable	126	126	Nepal (NMC)

Not applicable	127	127	Oman (NMC)

Not applicable	128	128	Pakistan (NMC)

Not applicable	129	129	Qatar (NMC)

Not applicable	130	130	Yemen (NMC)

Not applicable	131	131	Sri Lanka (NMC)

Not applicable	132	132	Tajikistan (NMC)

Not applicable	133	133	Turkmenistan (NMC)

Not applicable	134	134	United Arab Emirates (NMC)

Not applicable	135	135	Uzbekistan (NMC)

Not applicable	136	136	Viet Nam (NMC)

Not applicable	137–139	137–139	Reserved for other centres

Not applicable	140	140	Bolivia (Plurinational State of) (NMC)

Not applicable	141	141	Guyana (NMC)

Not applicable	142	142	Paraguay (NMC)

Not applicable	143	143	Suriname (NMC)

Not applicable	144	144	Uruguay (NMC)

Not applicable	145	145	French Guiana

Not applicable	146	146	Brazilian Navy Hydrographic Centre

Not applicable	147	147	National Commission on Space Activities (CONAE)
				– Argentina

Not applicable	148–149	148–149	Reserved for other centres

Not applicable	150	150	Antigua and Barbuda (NMC)

Not applicable	151	151	Bahamas (NMC)

Not applicable	152	152	Barbados (NMC)

Not applicable	153	153	Belize (NMC)

Not applicable	154	154	British Caribbean Territories Centre

Not applicable	155	155	San José

Not applicable	156	156	Cuba (NMC)

Not applicable	157	157	Dominica (NMC)

Not applicable	158	158	Dominican Republic (NMC)

Not applicable	159	159	El Salvador (NMC)

Not applicable	160	160	US NOAA/NESDIS

Not applicable	161	161	US NOAA Office of Oceanic and Atmospheric Research

Not applicable	162	162	Guatemala (NMC)

Not applicable	163	163	Haiti (NMC)

Not applicable	164	164	Honduras (NMC)

Not applicable	165	165	Jamaica (NMC)

Not applicable	166	166	Mexico City

Octet 5 in Section 1

Code figure	Code figure	of GRIB Edition 1

for F1F2	for F3F3F3	Octet 6 in Section 1

of BUFR Edition 3

Not applicable	167	167	Curaçao and Sint Maarten (NMC)

Not applicable	168	168	Nicaragua (NMC)

Not applicable	169	169	Panama (NMC)

Not applicable	170	170	Saint Lucia (NMC)

Not applicable	171	171	Trinidad and Tobago (NMC)

Not applicable	172	172	French Departments in RA IV

Not applicable	173	173	US National Aeronautics and Space Administration (NASA)

Not applicable	174	174	Integrated Science Data Management/Marine Environmental
				Data Service (ISDM/MEDS) – Canada

Not applicable	175	175	University Corporation for Atmospheric Research (UCAR)
				– United States

Not applicable	176	176	Cooperative Institute for Meteorological Satellite Studies
				(CIMSS) – United States

Not applicable	177	177	NOAA National Ocean Service – United States

Not applicable	178–189	178–189	Reserved for other centres

Not applicable	190	190	Cook Islands (NMC)

Not applicable	191	191	French Polynesia (NMC)

Not applicable	192	192	Tonga (NMC)

Not applicable	193	193	Vanuatu (NMC)

Not applicable	194	194	Brunei Darussalam (NMC)

Not applicable	195	195	Indonesia (NMC)

Not applicable	196	196	Kiribati (NMC)

Not applicable	197	197	Federated States of Micronesia (NMC)

Not applicable	198	198	New Caledonia (NMC)

Not applicable	199	199	Niue

Not applicable	200	200	Papua New Guinea (NMC)

Not applicable	201	201	Philippines (NMC)

Not applicable	202	202	Samoa (NMC)

Not applicable	203	203	Solomon Islands (NMC)

Not applicable	204	204	National Institute of Water and Atmospheric Research 					(NIWA – New Zealand)

Not applicable	205–209	205–209	Reserved

Not applicable	210	210	Frascati (ESA/ESRIN)

Not applicable	211	211	Lannion

Not applicable	212	212	Lisbon

Not applicable	213	213	Reykjavik

Not applicable	214	214	Madrid

Not applicable	215	215	Zurich

Not applicable	216	216	Service ARGOS – Toulouse

Not applicable	217	217	Bratislava

Not applicable	218	218	Budapest

Not applicable	219	219	Ljubljana

Not applicable	220	220	Warsaw

Not applicable	221	221	Zagreb

Not applicable	222	222	Albania (NMC)

Not applicable	223	223	Armenia (NMC)

Not applicable	224	224	Austria (NMC)

Not applicable	225	225	Azerbaijan (NMC)

Not applicable	226	226	Belarus (NMC)

Not applicable	227	227	Belgium (NMC)

Octet 5 in Section 1

Code figure	Code figure	of GRIB Edition 1

for F1F2	for F3F3F3	Octet 6 in Section 1

of BUFR Edition 3

Not applicable	228	228	Bosnia and Herzegovina (NMC)

Not applicable	229	229	Bulgaria (NMC)

Not applicable	230	230	Cyprus (NMC)

Not applicable	231	231	Estonia (NMC)

Not applicable	232	232	Georgia (NMC)

Not applicable	233	233	Dublin

Not applicable	234	234	Israel (NMC)

Not applicable	235	235	Jordan (NMC)

Not applicable	236	236	Latvia (NMC)

Not applicable	237	237	Lebanon (NMC)

Not applicable	238	238	Lithuania (NMC)

Not applicable	239	239	Luxembourg

Not applicable	240	240	Malta (NMC)

Not applicable	241	241	Monaco

Not applicable	242	242	Romania (NMC)

Not applicable	243	243	Syrian Arab Republic (NMC)

Not applicable	244	244	The former Yugoslav Republic of Macedonia (NMC)

Not applicable	245	245	Ukraine (NMC)

Not applicable	246	246	Republic of Moldova (NMC)

Not applicable	247	247	Operational Programme for the Exchange of weather RAdar
				information (OPERA) – EUMETNET

Not applicable	248	248	Montenegro (NMC)

Not applicable	249	249	Barcelona Dust Forecast Center

Not applicable	250	250	COnsortium for Small scale MOdelling (COSMO)

Not applicable	251	251	Meteorological Cooperation on Operational NWP

(MetCoOp)

Not applicable	252	252	Max Planck Institute for Meteorology (MPI-M)

Not applicable	253	253	Reserved for other centres

Not applicable	254	254	EUMETSAT Operation Centre

Not applicable	255	255	Missing value

Not applicable	256–999	Not applicable 	Not used

Notes:

(1)	The closed bracket sign ) indicates that the corresponding code figure is reserved for the previously named centre.

(2)	With GRIB or BUFR, to indicate whether the originating/generating centre is a sub-centre or not, the following procedure should be applied:

In GRIB edition 1, use octet 26 of section 1, or in BUFR edition 3, use octet 5 of section 1, with the following meaning:

Code figure

0	Not a sub-centre, the originating/generating centre is the centre defined by Octet 5 in section 1 of GRIB Edition 1, or by octet 6 in section 1 of BUFR edition 3.

1 to 254	Identifier of the sub-centre which is the originating/generating centre. The identifier of the sub-
centre is allocated by the associated centre which is defined by octet 5 in section 1 of GRIB edition 1, or octet 6 in section 1 of BUFR edition 3. The sub-centre identifiers should be supplied to the WMO Secretariat by the associated centre(s) for publication.

(3)	For the definitions of sub-centres provided to the WMO Secretariat, see Common Code table C–12.

COMMON CODE TABLE C–2:	Radiosonde/sounding system used

Code table 3685 – rara (Radiosonde/sounding system used) – for alphanumeric codes

Common Code table

Code table 0 02 011 (Radiosonde type) in BUFR

Date of	Code figure for	Code figure for

assignment of	rara	BUFR

number (necessary	(Code table	(Code table

after 30/06/2007)	3685)	 0 02 011)

Not applicable	00	0	Reserved

Before	01	1	iMet-1-BB (United States)

Not applicable	02	2	No radiosonde – passive target (e.g. reflector)

Not applicable	03	3	No radiosonde – active target (e.g. transponder)

Not applicable	04	4	No radiosonde – passive temperature-humidity profiler

Not applicable	05	5	No radiosonde – active temperature-humidity profiler

Not applicable	06	6	No radiosonde – radio-acoustic sounder

Before	07	7	iMet-1-AB (United States)

Not applicable	08	8	No radiosonde –... (reserved)

Not applicable	09	9	No radiosonde – system unknown or not specified

Before	10	10	VIZ type A pressure-commutated (United States)

Before	11	11	VIZ type B time-commutated (United States)

Before	12	12	RS SDC (Space Data Corporation – United States)

Before	13	13	Astor (no longer made – Australia)

Before	14	14	VIZ MARK I MICROSONDE (United States)

Before	15	15	EEC Company type 23 (United States)

Before	16	16	Elin (Austria)

Before	17	17	Graw G. (Germany)

Before	18	18	Graw DFM-06 (Germany)

Before	19	19	Graw M60 (Germany)

Before	20	20	Indian Meteorological Service MK3 (India)

Before	21	21	VIZ/Jin Yang MARK I MICROSONDE (Republic of Korea)

Before	22	22	Meisei RS2-80 (Japan)

Before	23	23	Mesural FMO 1950A (France)

Before	24	24	Mesural FMO 1945A (France)

Before	25	25	Mesural MH73A (France)

Before	26	26	Meteolabor Basora (Switzerland)

Before	27	27	AVK-MRZ (Russian Federation)

Before	28	28	Meteorit MARZ2-1 (Russian Federation)

Before	29	29	Meteorit MARZ2-2 (Russian Federation)

Before	30	30	Oki RS2-80 (Japan)

Before	31	31	VIZ/Valcom type A pressure-commutated (Canada)

Before	32	32	Shanghai Radio (China)

Before	33	33	UK Met Office MK3 (UK)

Before	34	34	Vinohrady (Czech Republic)

Before	35	35	Vaisala RS18 (Finland)

Before	36	36	Vaisala RS21 (Finland)

Date of	Code figure for	Code figure for

assignment of	rara	BUFR

number (necessary	(Code table	(Code table

after 30/06/2007)	 3685)	 0 02 011)

Before	37	37	Vaisala RS80 (Finland)

Before	38	38	VIZ LOCATE Loran-C (United States)

Before	39	39	Sprenger E076 (Germany)

Before	40	40	Sprenger E084 (Germany)

Before	41	41	Sprenger E085 (Germany)

Before	42	42	Sprenger E086 (Germany)

Before	43	43	AIR IS - 4A - 1680 (United States)

Before	44	44	AIR IS - 4A - 1680 X (United States)

Before	45	45	RS MSS (United States)

Before	46	46	AIR IS - 4A - 403 (United States)

Before	47	47	Meisei RS2-91 (Japan)

Before	48	48	VALCOM (Canada)

Before	49	49	VIZ MARK II (United States)

Before	50	50	Graw DFM-90 (Germany)

Before	51	51	VIZ-B2 (United States)

Before	52	52	Vaisala RS80-57H

Before	53	53	AVK-RF95 (Russian Federation)

Before	54	54	Graw DFM-97 (Germany)

Before	55	55	Meisei RS-01G (Japan)

Before	56	56	M2K2 (France)

Before	57	57	Modem M2K2-DC (France)

Before	58	58	AVK-BAR (Russian Federation)

Before	59	59	Modem M2K2-R 1680 MHz RDF radiosonde with pressure
				sensor chip (France)

Before	60	60	Vaisala RS80/MicroCora (Finland)

Before	61	61	Vaisala RS80/Loran/Digicora I, II or Marwin (Finland)

Before	62	62	Vaisala RS80/PCCora (Finland)

Before	63	63	Vaisala RS80/Star (Finland)

Before	64	64	Orbital Sciences Corporation, Space Data Division,
				transponder radiosonde, type 909-11-XX, where XX
				corresponds to the model of the instrument (United States)

Before	65	65	VIZ transponder radiosonde, model number 1499–520 (United 
				States)

Before	66	66	Vaisala RS80 /Autosonde (Finland)

Before	67	67	Vaisala RS80/Digicora III (Finland)

Before	68	68	AVK-RZM-2 (Russian Federation)

Before	69	69	MARL-A or Vektor-M-RZM-2 (Russian Federation)

Before	70	70	Vaisala RS92/Star (Finland)

Before	71	71	Vaisala RS90/Loran/Digicora I, II or Marwin (Finland)

Before	72	72	Vaisala RS90/PC-Cora (Finland)

Before	73	73	Vaisala RS90/Autosonde (Finland)

Before	74	74	Vaisala RS90/Star (Finland)

Date of	Code figure for	Code figure for

assignment of	rara	BUFR

number (necessary	(Code table	(Code table

after 30/06/2007)	 3685)	 0 02 011)

Before	75	75	AVK-MRZ-ARMA (Russian Federation)

Before	76	76	AVK-RF95-ARMA (Russian Federation)

Before	77	77	GEOLINK GPSonde GL98 (France)

Before	78	78	Vaisala RS90/Digicora III (Finland)

Before	79	79	Vaisala RS92/Digicora I, II or Marwin (Finland)

Before	80	80	Vaisala RS92/Digicora III (Finland)

Before	81	81	Vaisala RS92/Autosonde (Finland)

Before	82	82	Sippican MK2 GPS/STAR (United States) with rod thermistor,
				carbon element and derived pressure

Before	83	83	Sippican MK2 GPS/W9000 (United States) with rod thermistor,
				carbon element and derived pressure

Before	84	84	Sippican MARK II with chip thermistor, carbon element and derived

pressure from GPS height

Before	85	85	Sippican MARK IIA with chip thermistor, carbon element and
				derived pressure from GPS height

Before	86	86	Sippican MARK II with chip thermistor, pressure and carbon
				element

Before	87	87	Sippican MARK IIA with chip thermistor, pressure and carbon
				element

Before	88	88	MARL-A or Vektor-M-MRZ (Russian Federation)

Before	89	89	MARL-A or Vektor-M-BAR (Russian Federation)

Not applicable	90	90	Radiosonde not specified or unknown

Not applicable	91	91	Pressure only radiosonde

Not applicable	92	92	Pressure only radiosonde plus transponder

Not applicable	93	93	Pressure only radiosonde plus radar reflector

Not applicable	94	94	No pressure radiosonde plus transponder

Not applicable	95	95	No pressure radiosonde plus radar reflector

Not applicable	96	96	Descending radiosonde

Before	97	97	BAT-16P (South Africa)

Before	98	98	BAT-16G (South Africa)

Before	99	99	BAT-4G (South Africa)

Not available	100	Reserved for BUFR only

01	101	Not vacant

Not available	102–106	Reserved for BUFR only

07	107	Not vacant

Not available	108–109	Reserved for BUFR only

01/01/2008	10	110	Sippican LMS5 w/Chip Thermistor, duct mounted capacitance
				relative humidity sensor and derived pressure from GPS 					height

01/01/2008	11	111	Sippican LMS6 w/Chip Thermistor, external boom mounted
				capacitance relative humidity sensor, and derived pressure
				from GPS height

Needed	12	112	Vacant

15/09/2010	13	113	Vaisala RS92/MARWIN MW32 (Finland)

03/11/2011	14	114	Vaisala RS92/DigiCORA MW41 (Finland)

01/12/2011	15	115	PAZA-12M/Radiotheodolite-UL (Ukraine)

Date of	Code figure for	Code figure for

assignment of	rara	BUFR

number (necessary	(Code table	(Code table

after 30/06/2007)	 3685)	 0 02 011)

01/12/2011	16	116	PAZA-22/AVK-1 (Ukraine)

02/05/2012	17	117	Graw DFM-09 (Germany)

18	118	Not vacant

Needed	19	119	Vacant

20–21	120–121	Not vacant

02/05/2012	22	122	Meisei RS-11G GPS radiosonde 
				w/thermistor, capacitance relative 
				humidity sensor, and derived pressure 
				from GPS height (Japan)

03/11/2011	23	123	Vaisala RS41/DigiCORA MW41 (Finland)

03/11/2011	24	124	Vaisala RS41/AUTOSONDE (Finland)

03/11/2011	25	125	Vaisala RS41/MARWIN MW32 (Finland)

07/05/2014	26	126	Meteolabor SRS–C34/Argus 37 (Switzerland)

27	127	Not vacant

15/09/2011	28	128	AVK - AK2-02 (Russian Federation)

15/09/2011	29	129	MARL-A or Vektor-M - AK2-02 (Russian Federation)

01/01/2010	30	130	Meisei RS-06G (Japan)

03/11/2011	31	131	Taiyuan GTS1-1/GFE(L) (China )

03/11/2011	32	132	Shanghai GTS1/GFE(L) (China)

03/11/2011	33	133	Nanjing GTS1-2/GFE(L) (China)

Needed	34	134	Vacant

07/05/2014	35	135	Meisei iMS-100 GPS radiosonde w/thermistor sensor, capacitance

relative humidity sensor, and derived pressure from GPS height

(Japan)

Needed	36	136	Vacant

37	137	Not vacant

Needed	38–40	138–140	Vacant

03/11/2011	41	141	Vaisala RS41 with pressure derived from GPS height/
				DigiCORA MW41 (Finland)

03/11/2011	42	142	Vaisala RS41 with pressure derived from GPS height/
				AUTOSONDE (Finland)

07/05/2014	43	143	NanJing Daqiao XGP-3G (China)*

07/05/2014	44	144	TianJin HuaYunTianYi GTS(U)1 (China)*

07/05/2014	45	145	Beijing Changfeng CF-06 (China)*

07/05/2014	46	146	Shanghai Changwang GTS3 (China)*

47	147	Not vacant

02/05/2012	48	148	PAZA-22M/MARL-A

49	149	Not vacant

Needed	50	150	Vacant

51	151	Not vacant

03/11/2011	52	152	Vaisala RS92-NGP/Intermet IMS-2000 (United States)

53–59	153–159	Not vacant

____________

*	All GPS radiosondes are with thermistor, silicon piezoresistive pressure sensor or pressure derived from GPS height, capacitive relative humidity sensor and wind derived from GPS height.

Date of	Code figure for	Code figure for

assignment of	rara	BUFR

number (necessary	(Code table	(Code table

after 30/06/2007)	 3685)	 0 02 011)

Needed	60	160	Vacant

61	161	Not vacant

Needed	62–66	162–166	Vacant

67–72	167–172	Not vacant

Needed	73	173	Vacant

74–76	174–176	Not vacant

15/03/2010	77	177	Modem GPSonde M10 (France)

78–81	178–181	Not vacant

07/11/2012	82	182	Lockheed Martin LMS-6 w/chip
				thermistor; external boom mounted
				polymer capacitive relative humidity sensor;
				capacitive pressure sensor and GPS wind

07/11/2012	83	183	Vaisala RS92-D/Intermet IMS 1500
				w/silicon capacitive pressure sensor,
				capacitive wire temperature sensor, twin
				thin-film heated polymer capacitive relative
				humidity sensor and RDF wind

Needed	84	184	Vacant

85-89	185-189	Not vacant

Not available	190–196	Reserved for BUFR only

97–99	197–199	Not vacant

Not available	200–254	Reserved for BUFR only

255	Missing value

Notes:

(1)	References to countries in brackets indicate the manufacturing location rather than the country using the instrument.

(2)	Some of the radiosondes listed are no longer in use but are retained for archiving purposes.

(3)	The alphanumeric code format reports only 2 digits, and the first digit for BUFR is identified from the date: the first digit is 0 if the introduction of the radiosonde for observation was before 30 June 2007, or 1 otherwise. Entries in the second part of the table (after 99), which are declared “Vacant” can be used for new radiosondes because the 2-digit number was originally attributed to sondes, which are no longer used. This system has been adopted to accommodate reporting in TEMP traditional alphanumeric code format up to the time BUFR is fully used for radiosonding reports.

COMMON CODE TABLE C–3:	Instrument make and type for water temperature profile measurement with fall rate equation coefficients

Code table 1770 – IXIXIX (Instrument type for XBT, with fall rate equation coefficients)

Common Code table	 – for alphanumeric codes

Code table 0 22 067 (Instrument type for water temperature/salinity profile measurement) 
	in BUFR

Code figure for	Meaning

Code figure	BUFR	Instrument

for IXIXIX	(Code table 0 22 067)	make and type	Equation Coefficients

a	b

001	1	Sippican T-4	6.472	–2.16

002	2	Sippican T-4	6.691	–2.25

011	11	Sippican T-5	6.828	–1.82

021	21	Sippican Fast Deep	6.346	–1.82

031	31	Sippican T-6	6.472	–2.16

032	32	Sippican T-6	6.691	–2.25

041	41	Sippican T-7	6.472	–2.16

042	42	Sippican T-7	6.691	–2.25

051	51	Sippican Deep Blue	6.472	–2.16

052	52	Sippican Deep Blue	6.691	–2.25

061	61	Sippican T-10	6.301	–2.16

071	71	Sippican T-11	1.779	–0.255

081	81	Sippican AXBT (300 m probes)	1.52	0.0

201	201	TSK T-4	6.472	–2.16

202	202	TSK T-4	6.691	–2.25

211	211	TSK T-6	6.472	–2.16

212	212	TSK T-6	6.691	–2.25

221	221	TSK T-7	6.472	–2.16

222	222	TSK T-7	6.691	–2.25

231	231	TSK T-5	6.828	–1.82

241	241	TSK T-10	6.301	–2.16

251	251	TSK Deep Blue	6.472	–2.16

252	252	TSK Deep Blue	6.691	–2.25

261	261	TSK AXBT

401	401	Sparton XBT-1	6.301	–2.16

411	411	Sparton XBT-3	5.861	–0.0904

421	421	Sparton XBT-4	6.472	–2.16

431	431	Sparton XBT-5	6.828	–1.82

441	441	Sparton XBT-5DB	6.828	–1.82

451	451	Sparton XBT-6	6.472	–2.16

461	461	Sparton XBT-7	6.472	–2.16

462	462	Sparton XBT-7	6.705	–2.28

471	471	Sparton XBT-7DB	6.472	–2.16

481	481	Sparton XBT-10	6.301	–2.16

491	491	Sparton XBT-20	6.472	–2.16

501	501	Sparton XBT-20DB	6.472	–2.16

510	510	Sparton 536 AXBT	1.524	0

700	700	Sippican XCTD Standard

710	710	Sippican XCTD Deep

720	720	Sippican AXCTD

730	730	Sippican SXCTD

Code figure for	Meaning

Code figure	BUFR	Instrument

for IXIXIX	(Code table 0 22 067)	make and type	Equation Coefficients

a	b

741	741	TSK XCTD/XCTD-1	3.42543	–0.47

742	742	TSK XCTD-2	3.43898	–0.31

743	743	TSK XCTD-2F	3.43898	–0.31

744	744	TSK XCTD-3	5.07598	–0.72

745	745	TSK XCTD-4	3.68081	–0.47

751	751	TSK AXCTD

780	780	Sea-Bird SBE21 SEACAT 	Not applicable
			Thermosalinograph

781	781	Sea-Bird SBE45 MicroTSG 	Not applicable
			Thermosalinograph

800	800	Mechanical BT	Not applicable

810	810	Hydrocast	Not applicable

820	820	Thermistor chain	Not applicable

825	825	Temperature (sonic) and pressure	Not applicable
			probes

830	830	CTD	Not applicable

831	831	CTD-P-ALACE float	Not applicable

837	837	ARVOR_C, SBE conductivity sensor

838	838	ARVOR_D, SBE conductivity sensor

839	839	PROVOR–II, SBE conductivity sensor

840	840	PROVOR, no conductivity sensor	Not applicable

841	841	PROVOR, Sea-Bird conductivity	Not applicable

sensor

842	842	PROVOR, FSI conductivity sensor	Not applicable

843	843	Polar Ocean Profiling System

(POPS), PROVOR, SBE CTD

844	844	Profiling float, ARVOR, Sea-Bird

conductivity sensor

845	845	Webb Research, no conductivity	Not applicable

sensor

846	846	Webb Research, Sea-Bird	Not applicable

conductivity sensor

847	847	Webb Research, FSI conductivity	Not applicable

sensor

848	848	APEX–EM, SBE conductivity sensor

849	849	APEX_D, SBE conductivity sensor

850	850	SOLO, no conductivity sensor	Not applicable

851	851	SOLO, Sea-Bird conductivity	Not applicable

sensor

852	852	SOLO, FSI conductivity sensor	Not applicable

853	853	Profiling float, SOLO2 (SCRIPPS),

Sea-Bird conductivity sensor

854	854	S2A, SBE conductivity sensor

855	855	Profiling float, NINJA, no 	Not applicable

conductivity sensor

856	856	Profiling float, NINJA, SBE	Not applicable

conductivity sensor

857	857	Profiling float, NINJA, FSI 	Not applicable

conductivity sensor

858	858	Profiling float, NINJA, TSK 	Not applicable

conductivity sensor

Code figure for	Meaning

Code figure	BUFR	Instrument

for IXIXIX	(Code table 0 22 067)	make and type	Equation Coefficients

a	b

859	859	Profiling float, NEMO, no	Not applicable

conductivity sensor

860	860	Profiling float, NEMO, SBE	Not applicable

conductivity sensor

861	861	Profiling float, NEMO, FSI 	Not applicable

conductivity sensor

862	862	SOLO_D, SBE conductivity sensor

863	863	NAVIS–A, SBE conductivity sensor

864	864	NINJA_D, SBE conductivity sensor

865	865	NOVA, SBE conductivity sensor

866	866	ALAMO, No Conductivity sensor

867	867	ALAMO, RBR Conductivity sensor

868	868	ALAMO, SBE Conductivity sensor

869–899	866–899	Reserved

900	900	Sippican LMP-5 XBT	9.727	–0.0000473

901	901	Ice-tethered Profiler (ITP),

SBE CTD

902	902	Brooke ocean moving vessel profiler (MVP)

903	903	Seabird CTD

904	904	AML oceanographic CTD

905	905	Falmouth scientific CTD

906	906	Ocean sensors CTD

907	907	Valeport CTD

908	908	Ocean science MVP

909	909	Idronaut CTD

910	910	Seabird SBE38

911–994	911–994	Reserved

995	995	Instrument attached to	Not applicable

marine mammals

996	996	Instrument attached to animals 	Not applicable

other than marine mammals

997–999	997–999	Reserved

1000–1022	Reserved

1023	Missing value

Notes:

(1)	The depth is calculated from coefficients a and b and the time t as follows: z = at + 10–3 bt2

(2)	All unassigned numbers are reserved for future use.

(3)	The values of a and b are supplied for information only.

____________

COMMON CODE TABLE C–4:	Water temperature profile recorder types

Code table 4770 – XRXR (Recorder type) – for alphanumeric codes

Common Code table

Code table 0 22 068 (Water temperature profile recorder types) in BUFR

Code figure for

Code figure for	BUFR

XRXR  	(Code table 0 22 068)	Meaning

01	1	Sippican Strip Chart Recorder

02	2	Sippican MK2A/SSQ-61

03	3	Sippican MK-9

04	4	Sippican AN/BHQ-7/MK8

05	5	Sippican MK-12

06	6	Sippican MK-21

07	7	Sippican MK-8 Linear Recorder

08	8	Sippican MK-10

10	10	Sparton SOC BT/SV Processor Model 100

11	11	Lockheed-Sanders Model OL5005

20	20	ARGOS XBT-ST

21	21	CLS-ARGOS/Protecno XBT-ST Model-1

22	22	CLS-ARGOS/Protecno XBT-ST Model-2

30	30	BATHY Systems SA-810

31	31	Scripps Metrobyte Controller

32	32	Murayama Denki Z-60-16 III

33	33	Murayama Denki Z-60-16 II

34	34	Protecno ETSM2

35	35	Nautilus Marine Service NMS-XBT

40	40	TSK MK-2A

41	41	TSK MK-2S

42	42	TSK MK-30

43	43	TSK MK-30N

45	45	TSK MK-100

46	46	TSK MK-130 Compatible recorder for both XBT and XCTD

47	47	TSK MK-130A XCTD recorder

48	48	TSK AXBT RECEIVER MK-300

49	49	TSK MK-150/MK-150N Compatible recorder for both XBT and XCTD

50	50	JMA ASTOS

60	60	ARGOS communications, sampling on up transit

61	61	ARGOS communications, sampling on down transit

62	62	Orbcomm communications, sampling on up transit

63	63	Orbcomm communications, sampling on down transit

64	64	Iridium communications, sampling on up transit

65	65	Iridium communications, sampling on down transit

70	70	CSIRO Devil-1 XBT acquisition system

71	71	CSIRO Devil-2 XBT acquisition system

72	72	TURO/CSIRO Quoll XBT acquisition system

80	80	Applied Microsystems Ltd., MICRO-SVT&P

81	81	Sea Mammal Research Unit, Univ. St. Andrews, UK,

uncorrected salinity from a sea mammal mounted instrument

82	82	Sea Mammal Research Unit, Univ. St. Andrews, UK, corrected

salinity from a sea mammal mounted instrument

99	99	Unknown

127	Missing value

Note:	All unassigned numbers are reserved for future use.

COMMON CODE TABLE C–5:	Satellite identifier

I6I6I6 for alphanumeric codes

Common Code table	Code table 0 01 007 in BUFR

Code used in GRIB Edition 2

(EVEN DECILES INDICATE POLAR-ORBITING SATELLITES AND ODD DECILES INDICATE GEOSTATION-
ARY SATELLITES.)

Code figure for	Code figure for

Code figure for	BUFR	GRIB

I6I6I6   	(Code table 0 01 007)	Edition 2

000	0	0	Reserved

001–099: Numbers allocated to Europe

001	1	1	ERS 1

002	2	2	ERS 2

003	3	3	METOP-1 (Metop-B)

004	4	4	METOP-2 (Metop-A)

005	5	5	METOP-3 (Metop-C)

020	20	20	SPOT1

021	21	21	SPOT2

022	22	22	SPOT3

023	23	23	SPOT4

040	40	40	OERSTED

041	41	41	CHAMP

042	42	42	TerraSAR-X

043	43	43	TanDEM-X

044	44	44	PAZ

046	46	46	SMOS

047	47	47	CryoSat-2

048	48	48	AEOLUS

050	50	50	METEOSAT 3

051	51	51	METEOSAT 4

052	52	52	METEOSAT 5

053	53	53	METEOSAT 6

054	54	54	METEOSAT 7

055	55	55	METEOSAT 8

056	56	56	METEOSAT 9

057	57	57	METEOSAT 10

058	58	58	METEOSAT 1

059	59	59	METEOSAT 2

060	60	60	ENVISAT

061	61	61	Sentinel 3A

070	70	70	METEOSAT 11

100–199: Numbers allocated to Japan

120	120	120	ADEOS

121	121	121	ADEOS II

122	122	122	GCOM-W1

140	140	140	GOSAT

150	150	150	GMS 3

151	151	151	GMS 4

Code figure for	Code figure for

Code figure for	BUFR	GRIB

I6I6I6 	(Code table 0 01 007)	 Edition 2

152	152	152	GMS 5

153	153	153	GMS

154	154	154	GMS-2

171	171	171	MTSAT-1R

172	172	172	MTSAT-2

173	173	173	Himawari-8

174	174	174	Himawari-9

200–299: Numbers allocated to the United States

200	200	200	NOAA 8

201	201	201	NOAA 9

202	202	202	NOAA 10

203	203	203	NOAA 11

204	204	204	NOAA 12

205	205	205	NOAA 14

206	206	206	NOAA 15

207	207	207	NOAA 16

208	208	208	NOAA 17

209	209	209	NOAA 18

220	220	220	LANDSAT 5

221	221	221	LANDSAT 4

222	222	222	LANDSAT 7

223	223	223	NOAA 19

224	224	224	NPP

240	240	240	DMSP 7

241	241	241	DMSP 8

242	242	242	DMSP 9

243	243	243	DMSP 10

244	244	244	DMSP 11

245	245	245	DMSP 12

246	246	246	DMSP 13

247	247	247	DMSP 14

248	248	248	DMSP 15

249	249	249	DMSP 16

250	250	250	GOES 6

251	251	251	GOES 7

252	252	252	GOES 8

253	253	253	GOES 9

254	254	254	GOES 10

255	255	255	GOES 11

256	256	256	GOES 12

257	257	257	GOES 13

258	258	258	GOES 14

259	259	259	GOES 15

260	260	260	JASON 1

Code figure for	Code figure for

Code figure for	BUFR	GRIB

I6I6I6 	(Code table 0 01 007)	 Edition 2

261	261	261	JASON 2

281	281	281	QUIKSCAT

282	282	282	TRMM

283	283	283	CORIOLIS

285	285	285	DMSP17

286	286	286	DMSP18

287	287	287	DMSP-19

288	288	288	GPM-core

300–399: Numbers allocated to the Russian Federation

310	310	310	GOMS 1

311	311	311	GOMS 2

320	320	320	METEOR 2-21

321	321	321	METEOR 3-5

322	322	322	METEOR 3M-1

323	323	323	METEOR 3M-2

341	341	341	RESURS 01-4

400–499: Numbers allocated to India

410	410	410	KALPANA-1

421	421	421	Oceansat-2

430	430	430	INSAT 1B

431	431	431	INSAT 1C

432	432	432	INSAT 1D

440	440	440	Megha-Tropiques

441	441	441	SARAL

450	450	450	INSAT 2A

451	451	451	INSAT 2B

452	452	452	INSAT 2E

470	470	470	INSAT 3A

471	471	471	INSAT 3D

472	472	472	INSAT 3E

500–599: Numbers allocated to China

500	500	500	FY-1C

501	501	501	FY-1D

510	510	510	FY-2

512	512	512	FY-2B

513	513	513	FY-2C

514	514	514	FY-2D

515	515	515	FY-2E

516	516	516	FY-2F

517	517	517	FY-2G

520	520	520	FY-3A

521	521	521	FY-3B

522	522	522	FY-3C

600–699: Numbers allocated to Europe

Code figure for	Code figure for

Code figure for	BUFR	GRIB

I6I6I6 	(Code table 0 01 007)	 Edition 2

700–799: Numbers allocated to the United States

700	700	700	TIROS M (ITOS 1)

701	701	701	NOAA 1

702	702	702	NOAA 2

703	703	703	NOAA 3

704	704	704	NOAA 4

705	705	705	NOAA 5

706	706	706	NOAA 6

707	707	707	NOAA 7

708	708	708	TIROS-N

710	710	710	GOES (SMS 1)

711	711	711	GOES (SMS 2)

720	720	720	TOPEX

721	721	721	GFO (GEOSAT follow on)

722	722	722	GRACE A

723	723	723	GRACE B

731	731	731	GOES 1

732	732	732	GOES 2

733	733	733	GOES 3

734	734	734	GOES 4

735	735	735	GOES 5

740	740	740	COSMIC-1

741	741	741	COSMIC-2

742	742	742	COSMIC-3

743	743	743	COSMIC-4

744	744	744	COSMIC-5

745	745	745	COSMIC-6

763	763	763	NIMBUS 3

764	764	764	NIMBUS 4

765	765	765	NIMBUS 5

766	766	766	NIMBUS 6

767	767	767	NIMBUS 7

780	780	780	ERBS

781	781	781	UARS

782	782	782	EARTH PROBE

783	783	783	TERRA

784	784	784	AQUA

785	785	785	AURA

786	786	786	C/NOFS

787	787	787	CALIPSO

788	788	788	CloudSat

800–849 Numbers allocated to other satellite operators

800	800	800	SUNSAT

810	810	810	COMS-1

811	811	811	COMS-2

Code figure for	Code figure for

Code figure for	BUFR	GRIB

I6I6I6 	(Code table 0 01 007)	 Edition 2

820	820	820	SAC-C

821	821	821	SAC-D

825	825	825	KOMPSAT-5

850	850	850	Combination of TERRA and AQUA

851	851	851	Combination of NOAA 16 to NOAA 19

852	852	852	Combination of Metop-1 to Metop-3

853	853	853	Combination of METEOSAT and DMSP

854	854	854	Non-specific mixture of geostationary and low

Earth-orbiting satellites

870–998	870–998	870–998	Reserved

999 Missing value	999–1022	999–65534	Reserved

1023	65535	Missing value

Note:	Within the ranges 000 to 849 and 870 to 998, even deciles indicate polar orbiting satellites and odd deciles indicate geostationary satellites. The range from 850 to 869 shall be used to indicate combinations of satellites, so the aforementioned decile rule does not apply to values in this range.

COMMON CODE TABLE C–6: List of units for TDCFs

| Code figure |  |  | Conventional abbreviation |  | Abbreviation in IA5/ASCII (5) |  | Abbreviation in ITA2 (5) |  | Definition in base units (2) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Base SI units (1) |  |  |  |  |  |  |  |  |  |
| 001 | metre |  | m |  | m |  | M |  |  |  |
| 002 | kilogram |  | kg |  | kg |  | KG |  |  |  |
| 003 | second |  | s |  | s |  | S |  |  |  |
| 004 | ampere |  | A |  | A |  | A |  |  |  |
| 005 | kelvin |  | K |  | K |  | K |  |  |  |
| 006 | mole |  | mol |  | mol |  | MOL |  |  |  |
| 007 | candela |  | cd |  | cd |  | CD |  |  |  |
|  | Supplementary SI Units (1) |  |  |  |  |  |  |  |  |  |
| 021 | radian |  | rad |  | rad |  | RAD |  |  |  |
| 022 | steradian |  | sr |  | sr |  | SR |  |  |  |
|  | Derived SI Units with special names (1) |  |  |  |  |  |  |  |  |  |
| 030 | hertz |  | Hz |  | Hz |  | HZ |  | s–1 |  |
| 031 | newton |  | N |  | N |  | N |  | kg m s–2 |  |
| 032 | pascal |  | Pa |  | Pa |  | PAL |  | kg m–1 s–2 |  |
| 033 | joule |  | J |  | J |  | J |  | kg m2 s–2 |  |
| 034 | watt |  | W |  | W |  | W |  | kg m2 s–3 |  |
| 035 | coulomb |  | C |  | C |  | C |  | A s |  |
| 036 | volt |  | V |  | V |  | V |  | kg m2 s–3 A–1 |  |
| 037 | farad |  | F |  | F |  | F |  | kg–1 m–2 s4 A2 |  |
| 038 | ohm |  | Ω |  | Ohm |  | OHM |  | kg m2 s–3 A–2 |  |
| 039 | siemens |  | S |  | S |  | SIE |  | kg–1 m–2 s3 A2 |  |
| 040 | weber |  | Wb |  | Wb |  | WB |  | kg m2 s–2 A–1 |  |
| 041 | tesla |  | T |  | T |  | T |  | kg s–2 A–1 |  |
| 042 | henry |  | H |  | H |  | H |  | kg m2 s–2 A–2 |  |
| 060 | degree Celsius |  | °C |  | Cel |  | CEL |  | K+273.15 |  |
| 070 | lumen |  | lm |  | lm |  | LM |  | cd sr |  |
| 071 | lux |  | lx |  | lx |  | LX |  | cd sr m–2 |  |
| 080 | becquerel |  | Bq |  | Bq |  | BQ |  | s–1 |  |
| 081 | gray |  | Gy |  | Gy |  | GY |  | m2 s–2 |  |
| 082 | sievert |  | Sv |  | Sv |  | SV |  | m2 s–2 |  |
|  | SI Unit prefixes (1) (3) (4) |  |  |  |  |  |  |  |  |  |
| no | (yotta) |  | (Y) |  | (Y) |  | (Y) |  |  |  |
| no | (zetta) |  | (Z) |  | (Z) |  | (Z) |  |  |  |
| no | exa |  | E |  | E |  | E |  |  |  |
| no | peta |  | P |  | P |  | PE |  |  |  |
| no | tera |  | T |  | T |  | T |  |  |  |
| no | giga |  | G |  | G |  | G |  |  |  |
| no | mega |  | M |  | M |  | MA |  |  |  |
| no | kilo |  | k |  | k |  | K |  |  |  |
| no | hecto |  | h |  | h |  | H |  |  |  |
| no | deca |  | da |  | da |  | DA |  |  |  |
| no | deci |  | d |  | d |  | D |  |  |  |
| no | centi |  | c |  | c |  | C |  |  |  |
| no | milli |  | m |  | m |  | M |  |  |  |
| no | micro |  | μ |  | u |  | U |  |  |  |
| Code figure |  |  | Conventional abbreviation |  | Abbreviation in IA5/ASCII (5) |  | Abbreviation in ITA2 (5) |  | Definition in base units (2) |  |
| no | nano |  | n |  | n |  | N |  |  |  |
| no | pico |  | p |  | p |  | P |  |  |  |
| no | femto |  | f |  | f |  | F |  |  |  |
| no | atto |  | a |  | a |  | A |  |  |  |
| no | (zepto) |  | (z) |  | (z) |  |  |  |  |  |
| no | (yocto) |  | (y) |  | (y) |  |  |  |  |  |
|  | Other, non-SI, units recognized by CGPM (4) |  |  |  |  |  |  |  |  |  |
| 110 | degree (angle) |  | ° |  | deg |  | DEG |  |  |  |
| 111 | minute (angle) |  | ' |  | ' |  | MNT |  |  |  |
| 112 | second (angle) |  | " |  | " |  | SEC |  |  |  |
| 120 | litre |  | l or L |  | l or L |  | L |  |  |  |
| 130 | minute (time) |  | min |  | min |  | MIN |  |  |  |
| 131 | hour |  | h |  | h |  | HR |  |  |  |
| 132 | day |  | d |  | d |  | D |  |  |  |
| 150 | tonne |  | t |  | t |  | TNE |  |  |  |
| 160 | electron volt |  | eV |  | eV |  | EV |  |  |  |
| 161 | atomic mass unit |  | u |  | u |  | U |  |  |  |
| 170 | astronomic unit |  | AU |  | AU |  | ASU |  |  |  |
| 171 | parsec |  | pc |  | pc |  | PRS |  |  |  |
|  | Non-SI Units tolerated because of widespread use |  |  |  |  |  |  |  |  |  |
| 200 | nautical mile |  |  |  |  |  |  |  |  |  |
| 201 | knot |  | kt |  | kt |  | KT |  |  |  |
| 210 | decibel (6) |  | dB |  | dB |  | DB |  |  |  |
| 220 | hectare |  | ha |  | ha |  | HAR |  |  |  |
| 230 | week |  |  |  |  |  |  |  |  |  |
| 231 | year |  | a |  | a |  | ANN |  |  |  |
|  | Other Units as used by WMO (7) |  |  |  |  |  |  |  |  |  |
| 300 | per cent |  | % |  | % |  | PERCENT |  |  |  |
| 301 | parts per thousand |  | ‰ |  | 0/00 |  | PERTHOU |  |  |  |
| 310 | eighths of cloud |  | okta |  | okta |  | OKTA |  |  |  |
| 320 | degrees true |  | ° |  | deg |  | DEG |  |  |  |
| 321 | degrees per second |  | degree/s |  | deg/s |  | DEG/S |  |  |  |
| 350 | degrees Celsius (8) |  | °C |  | C |  | C |  |  |  |
| 351 | degrees Celsius per metre |  | °C/m |  | C/m |  | C/M |  |  |  |
| 352 | degrees Celsius per 100 metres |  | °C/100 m |  | C/100 m |  | C/100 M |  |  |  |
| 360 | Dobson Unit (9) |  | DU |  | DU |  | DU |  |  |  |
| 430 | month |  | mon |  | mon |  | MON |  |  |  |
| 441 | per second (same as hertz) |  | s–1 |  | /s |  | /S |  |  |  |
| 442 | per second squared |  | s–2 |  | s–2 |  |  |  |  |  |
| 501 | knots per 1000 metres |  | kt/1000 m |  | kt/km |  | KT/KM |  |  |  |
| 510 | foot |  | ft |  | ft |  | FT |  |  |  |
| 511 | inch |  | in |  | in |  | IN |  |  |  |
| 520 | decipascals per second (microbar per second) |  | dPa s–1 |  | dPa/s |  | DPAL/S |  |  |  |
| 521 | centibars per second |  | cb s–1 |  | cb/s |  | CB/S |  |  |  |
| 522 | centibars per 12 hours |  | cb/12 h |  | cb/12 h |  | CB/12 HR |  |  |  |
| 523 | dekapascal |  | daPa |  | daPa |  | DAPAL |  |  |  |
| Code figure |  | Conventional abbreviation |  | Abbreviation in IA5/ASCII (5) |  | Abbreviation in ITA2 (5) |  | Definition in base units (2) |  |
| 530 | hectopascal |  | hPa |  | hPa |  | HPAL |  |  |  |
| 531 | hectopascals per second |  | hPa s–1 |  | hPa/s |  | HPAL/S |  |  |  |
| 532 | hectopascals per hour |  | hPa h–1 |  | hPa/h |  | HPAL/HR |  |  |  |
| 533 | hectopascals per 3 hours |  | hPa/3 h |  | hPa/3 h |  | HPAL/3 HR |  |  |  |
| 535 | nanobar = hPa 10–6 |  | nbar |  | nbar |  | NBAR |  |  |  |
| 620 | grams per kilogram |  | g kg–1 |  | g/kg |  | G/KG |  |  |  |
| 621 | grams per kilogram per second |  | g kg–1 s–1 |  | g kg–1 s–1 |  |  |  |  |  |
| 622 | kilograms per kilogram |  | kg kg–1 |  | kg/kg |  | KG/KG |  |  |  |
| 623 | kilograms per kilogram per second |  | kg kg–1 s–1 |  | kg kg–1 s–1 |  |  |  |  |  |
| 624 | kilograms per square metre |  | kg m–2 |  | kg m–2 |  |  |  |  |  |
| 630 | acceleration due to gravity |  | g |  | g |  |  |  |  |  |
| 631 | geopotential metre |  | gpm |  | gpm |  |  |  |  |  |
| 710 | millimetre |  | mm |  | mm |  | MM |  |  |  |
| 711 | millimetres per second |  | mm s–1 |  | mm/s |  | MM/S |  |  |  |
| 712 | millimetres per hour |  | mm h–1 |  | mm/h |  | MM/HR |  |  |  |
| 713 | millimetres to the sixth power per cubic metre |  | mm6 m–3 |  | mm6 m–3 |  |  |  |  |  |
| 715 | centimetre |  | cm |  | cm |  | CM |  |  |  |
| 716 | centimetres per second |  | cm s–1 |  | cm/s |  | CM/S |  |  |  |
| 717 | centimetres per hour |  | cm h–1 |  | cm/h |  | CM/HR |  |  |  |
| 720 | decimetre |  | dm |  | dm |  | DM |  |  |  |
| 731 | metres per second |  | m s–1 |  | m/s |  | M/S |  |  |  |
| 732 | metres per second per metre |  | m s–1/m |  | m s–1/m |  |  |  |  |  |
| 733 | metres per second per 1000 metres |  | m s–1/1000 m |  | m s–1/km |  |  |  |  |  |
| 734 | square metres |  | m2 |  | m2 |  | M2 |  |  |  |
| 735 | square metres per second |  | m2 s–1 |  | m2/s |  | M2/S |  |  |  |
| 740 | kilometre |  | km |  | km |  | KM |  |  |  |
| 741 | kilometres per hour |  | km h–1 |  | km/h |  | KM/HR |  |  |  |
| 742 | kilometres per day |  | km/d |  | km/d |  | KM/D |  |  |  |
| 743 | per metre |  | m–1 |  | m–1 |  | /M |  |  |  |
| 750 | becquerels per litre |  | Bq l–1 |  | Bq/l |  | BQ/L |  |  |  |
| 751 | becquerels per square metre |  | Bq m–2 |  | Bq m–2 |  | BQ/M2 |  |  |  |
| 752 | becquerels per cubic metre |  | Bq m–3 |  | Bq m–3 |  | BQ/M3 |  |  |  |
| 753 | millisievert |  | mSv |  | mSv |  | MSV |  |  |  |
| 760 | metres per second squared |  | m s–2 |  | m s–2 |  |  |  |  |  |
| 761 | square metres second |  | m2 s |  | m2 s |  |  |  |  |  |
| 762 | square metres per second squared |  | m2 s–2 |  | m2 s–2 |  |  |  |  |  |
| 763 | square metres per radian second |  | m2 rad–1 s |  | m2 rad–1 s |  |  |  |  |  |
| 764 | square metres per hertz |  | m2 Hz–1 |  | m2/Hz |  |  |  |  |  |
| 765 | cubic metres |  | m3 |  | m3 |  |  |  |  |  |
| 766 | cubic metres per second |  | m3 s–1 |  | m3/s |  |  |  |  |  |
| 767 | cubic metres per cubic metre |  | m3 m–3 |  | m3 m–3 |  |  |  |  |  |
| 768 | metres to the fourth power |  | m4 |  | m4 |  |  |  |  |  |
| 769 | metres to the two thirds power per second |  | m2/3 s–1 |  | m2/3 s–1 |  |  |  |  |  |
| 772 | logarithm per metre |  | log (m–1) |  | log (m–1) |  |  |  |  |  |
| 773 | logarithm per square metre |  | log (m–2) |  | log (m–2) |  |  |  |  |  |
| 775 | kilograms per metre |  | kg m–1 |  | kg/m |  |  |  |  |  |

| Code figure |  | Conventional abbreviation |  | Abbreviation in IA5/ASCII (5) |  | Abbreviation in ITA2 (5) |  | Definition in base units (2) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 776 | kilograms per square metre per second |  | kg m–2 s–1 |  | kg m–2 s–1 |  |  |  |  |  |
| 777 | kilograms per cubic metre |  | kg m–3 |  | kg m–3 |  |  |  |  |  |
| 778 | per square kilogram per second |  | kg–2 s–1 |  | kg–2 s–1 |  |  |  |  |  |
| 779 | seconds per metre |  | s m–1 |  | s/m |  |  |  |  |  |
| 785 | kelvin metres per second |  | K m s–1 |  | K m s–1 |  |  |  |  |  |
| 786 | kelvins per metre |  | K m–1 |  | K/m |  |  |  |  |  |
| 787 | kelvin square metres per kilogram per second |  | K m2 kg–1 s–1 |  | K m2 kg–1 s–1 |  |  |  |  |  |
| 788 | moles per mole |  | mol mol–1 |  | mol/mol |  |  |  |  |  |
| 790 | radians per metre |  | rad m–1 |  | rad/m |  |  |  |  |  |
| 795 | newtons per square metre |  | N m–2 |  | N m–2 |  |  |  |  |  |
| 800 | pascals per second |  | Pa s–1 |  | Pa/s |  |  |  |  |  |
| 801 | kilopascal |  | kPa |  | kPa |  |  |  |  |  |
| 805 | joules per square metre |  | J m–2 |  | J m–2 |  |  |  |  |  |
| 806 | joules per kilogram |  | J kg–1 |  | J/kg |  |  |  |  |  |
| 810 | watts per metre per steradian |  | W m–1 sr–1 |  | W m–1 sr–1 |  |  |  |  |  |
| 811 | watts per square metre |  | W m–2 |  | W m–2 |  |  |  |  |  |
| 812 | watts per square metre per steradian |  | W m–2 sr–1 |  | W m–2 sr–1 |  |  |  |  |  |
| 813 | watts per square metre per steradian centimetre |  | W m–2 sr–1 cm |  | W m–2 sr–1 cm |  |  |  |  |  |
| 814 | watts per square metre per steradian metre |  | W m–2 sr–1 m |  | W m–2 sr–1 m |  |  |  |  |  |
| 815 | watts per cubic metre per steradian |  | W m–3 sr–1 |  | W m–3 sr–1 |  |  |  |  |  |
| 820 | siemens per metre |  | S m–1 |  | S/m |  |  |  |  |  |
| 825 | square degrees |  | degree2 |  | deg2 |  |  |  |  |  |
| 830 | becquerel seconds per cubic metre |  | Bq s m–3 |  | Bq s m–3 |  |  |  |  |  |
| 835 | decibels per metre |  | dB m–1 |  | dB/m |  |  |  |  |  |
| 836 | decibels per degree |  | dB degree–1 |  | dB/deg |  |  |  |  |  |
| 841 | pH unit |  | pH unit |  | pH unit |  |  |  |  |  |
| 842 | N units |  | N units |  | N units |  |  |  |  |  |

Notes:

(1)	The international system of units, Système International d'Unités (SI), was established by the eleventh General Conference on Weights and Measures in 1960, and extended at the 1980 Conference. There are seven base units, two dimensionless supplementary units and a set of prefixes for decimal scaling. These may be combined to give compound units. Some compound units have special names, and are called derived Units.

(2)	When documenting compound SI units, each symbol for each base unit has been separated from the others by a space. There is no space between the unit and any prefix or exponent. Any prefix establishes a new unit to which any exponent applies (e.g. km2  = (km)2 = m6 not k(m2) = m5). Prefixes must be in the case specified. The full name of the unit must not start with an upper case letter. If the solidus (/) is used, there must be only one. There is no space before or after it.

(3)	Prefixes beyond exa and atto have been proposed but not yet adopted. Use of the prefixes hecto, deca, deci and centi is discouraged.

(4)	Prefixes generally should not be used with units having non-decimal multiples and sub-multiples, such as units of time and angle, or with knots and nautical miles.

(5)	Non-WMO abbreviations with limited character sets taken from ISO 2955-1983. Other abbreviations try to be consistent with this.

(6)	The decibel is one tenth of a bel, which is the decimal logarithm of a ratio of two powers. Frequently, suffixes are supplied to indicate information about one of the quantities in the ratio, such as dB(mW), dBm, dBZ, dBW, dBmW, dB(μV/m). It is recommended that only dB is used, with the full meaning of the ratio explained, including reference levels.

(7)	This list consists of the units not mentioned previously that occur in existing WMO Manuals.

(8)	The abbreviation for degrees Celsius proposed for WMO use, C, could be confused with Coulombs. In this case, Amperes second should be used instead.

(9)	Dobson Unit = DU. One Dobson Unit corresponds to a layer of 0.01 mm of pure ozone, if the whole column of atmosphere were compressed at P = 1013 hPa and T = 0 °C.

COMMON CODE TABLE C–7:	Tracking technique/status of system used

Code table 3872 – sasa for alphanumeric codes

Common Code table

Code table 0 02 014 in BUFR

Code figure for

Code figure for	BUFR

sasa	(Code table 0 02 014)

00	0	No wind finding

01	1	Automatic with auxiliary optical direction finding

02	2	Automatic with auxiliary radio direction finding

03	3	Automatic with auxiliary ranging

04	4	Not used

05	5	Automatic with multiple VLF-Omega signals

06	6	Automatic cross chain Loran-C

07	7	Automatic with auxiliary wind profiler

08	8	Automatic satellite navigation

09–18	9–18	Reserved

19	19	Tracking technique not specified

TRACKING TECHNIQUES/STATUS OF ASAP SYSTEM

STATUS OF SHIP SYSTEM

20	20	Vessel stopped

21	21	Vessel diverted from original destination

22	22	Vessel's arrival delayed

23	23	Container damaged

24	24	Power failure to container

25–28	25–28	Reserved for future use

29	29	Other problems

SOUNDING SYSTEM

30	30	Major power problems

31	31	UPS inoperative

32	32	Receiver hardware problems

33	33	Receiver software problems

34	34	Processor hardware problems

35	35	Processor software problems

36	36	NAVAID system damaged

37	37	Shortage of lifting gas

38	38	Reserved

39	39	Other problems

LAUNCH FACILITIES

40	40	Mechanical defect

41	41	Material defect (hand launcher)

42	42	Power failure

43	43	Control failure

44	44	Pneumatic/hydraulic failure

Code figure for

Code figure for	BUFR

sasa	(Code table 0 02 014)

45	45	Other problems

46	46	Compressor problems

47	47	Balloon problems

48	48	Balloon release problems

49	49	Launcher damaged

DATA ACQUISITION SYSTEM

50	50	R/S receiver antenna defect

51	51	NAVAID antenna defect

52	52	R/S receiver cabling (antenna) defect

53	53	NAVAID antenna cabling defect

54–58	54–58	Reserved

59	59	Other problems

COMMUNICATIONS

60	60	ASAP communications defect

61	61	Communications facility rejected data

62	62	No power at transmitting antenna

63	63	Antenna cable broken

64	64	Antenna cable defect

65	65	Message transmitted power below normal

66–68	66–68	Reserved

69	69	Other problems

70	70	All systems in normal operation

71–98	71–98	Reserved

99	99	Status of system and its components not specified

Not available	100–126	Reserved

Not available	127	Missing value

COMMON CODE TABLE C–8:	Satellite instruments

Common Code table	Code table 0 02 019 in BUFR

Code	Agency	Type	Instrument short name	Instrument long name

10	BNSC	Radiometer	AATSR	Advanced along track scanning radiometer

11	BNSC	Radiometer	ATSR	Along track scanning radiometer

12	BNSC	Radiometer	ATSR-2	Along track scanning radiometer-2

13	BNSC	Radiometer	MWR	Microwave radiometer

30	CNES	Communications	ARGOS

40	CNES	Lidar	Laser reflectors

41	CNES	Lidar	DORIS	Doppler orbitography and radio-positioning

integrated by satellite

42	CNES	Lidar	DORIS-NG	Doppler orbitography and radio-positioning

integrated by satellite-NG

47	CNES	Radar altimeter	POSEIDON-1 	Positioning ocean, solid Earth, ice dynamics

(SSALT1)	orbiting navigator (single frequency solid state

radar altimeter)

48	CNES	Radar altimeter	POSEIDON-2 	Positioning ocean, solid Earth, ice dynamics

(SSALT2)	orbiting navigator (double frequency solid

state radar altimeter)

49	CNES	Radar altimeter	POSEIDON-3 	Positioning ocean, solid Earth, ice dynamics 
				(SSALT3)	orbiting navigator (double frequency solid

state radar altimeter)

50	CNES	Imaging radiometer	ATSR/M	ATSR/M

51	CNES	High-resolution 	HRG

optical imager

52	CNES	Radiometer	HRV	High-resolution visible

53	CNES	Radiometer	HRVIR	High-resolution visible and infrared

54	CNES	Radiometer	ScaRaB/MV2	Scanner for Earth's radiation budget

55	CNES	Radiometer	POLDER	POLDER

56	CNES	Imaging multi-	IIR	Imaging Infrared Radiometer
			spectral radiometer

60	CNES	Spectrometer	VEGETATION	VEGETATION

61	CNES	Spectrometer	WINDII	WINDII

62	CNES	Altimeter	AltiKa	Ka band Radar Altimeter

80	CSA	Communications	RADARSAT DTT

81	CSA	Communications	RADARSAT TTC

85	CSA	Radar	SAR (CSA)	Synthetic aperture radar (CSA)

90	CSA	Radiometer	MOPITT	Measurements of pollution in the troposphere

91	CSA	Atmospheric 	OSIRIS	Optical spectrograph and Infrared imaging

chemistry instrument		system

97	CSIRO	Radiometer	Panchromatic imager

98	CRCSS	Atmospheric 	GPS receiver

temperature and

humidity sounder

102	DLR	Radiometer	CHAMP GPS sounder	GPS turborogue space receiver (TRSR)

103	DLR	Radiometer	IGOR	Integrated GPS and Occultation Receiver

116	DLR	Magnetometer	CHAMP gravity package	STAR accelerometer

(Accelerometer+GPS)

Code	Agency	Type	Instrument short name	Instrument long name

117	DLR	Magnetometer	CHAMP magnetometry	Overhauser magnetometer (OVM) and

package (1 scalar+	fluxgate magnetometer (FGM)

2 vector magnetometer)

120	ESA	Communications	ENVISAT Comms	Communications package on ENVISAT

121	ESA	Communications	ERS Comms	Communication package for ERS

130	ESA	Lidar	ALADIN	Atmospheric laser Doppler instrument

131	ESA	Lidar	ATLID	Atmospheric lidar

140	ESA	Radar	AMI/SAR/image	Active microwave instrumentation image 
					mode

141	ESA	Radar	AMI/SAR/wave	Active microwave instrumentation wave mode

142	ESA	Radar	AMI/scatterometer	Active microwave instrumentation wind mode

143	ESA	Radar	ASAR	ASAR

144	ESA	Imaging microwave	ASAR	Advanced synthetic aperture radar
					(image mode)

145	ESA	Imaging microwave	ASAR	Advanced synthetic aperture radar

(wave mode)

146	ESA	Cloud profile and	CPR	Cloud radar

rain radar

147	ESA	Radar	RA-2/MWR	Radar altimeter-2

148	ESA	Radar	RA/MWR	Radar altimeter

150	ESA	Scatterometer	SCATTEROMETER	Scatterometer

161	ESA	Radiometer	MIPAS	Michelson interferometric passive atmosphere

sounder

162	ESA	Imaging multi-	MWR-2	Microwave radiometer-2

spectral radiometer
			(passive microwave)

163	ESA	Atmospheric 	SOPRANO	Sub-milimetre observation of processes in the

chemistry instrument		absorption noteworthy for ozone

170	ESA	Atmospheric 	GOME	Global ozone monitoring experiment

chemistry instrument

172	ESA	Spectrometer	GOMOS	Global ozone monitoring by occultation of 
					stars

174	ESA	Spectrometer	MERIS	Medium resolution imaging spectrometer

175	ESA	Spectrometer	SCIAMACHY	Scanning imaging absorption spectrometer for

atmospheric cartography

176	ESA	Radiometer	MIRAS	Microwave imaging radiometer using aperture
					synthesis

177	ESA	Radar Altimeter	SIRAL	SAR/Interferometric Radar Altimeter

178	ESA	Radar altimeter	SRAL	Synthetic aperture radar altimeter

179	ESA	Moderate resolution 	OLCI	Ocean and land colour imager
			optical imager

180	ESA	Moderate resolution 	SLSTR	Sea and land surface temperature radiometer
			optical imager

181	EUMETSAT	Communications	METEOSAT Comms	Communications package for METEOSAT

182	EUMETSAT	Communications	MSG Comms	Communications package for MSG

190	ESA/	Scatterometer	ASCAT	Advanced scatterometer
		EUMETSAT

200	EUMETSAT	Radiometer	GERB	Geostationary Earth radiation budget

202	ESA/	Radiometer	GRAS	GNSS receiver for atmospheric sounding
		EUMETSAT

Code	Agency	Type	Instrument short name	Instrument long name

203	EUMETSAT	Radiometer	MHS	Microwave humidity sounder

205	EUMETSAT	Radiometer	MVIRI	METEOSAT visible and infrared imager

207	EUMETSAT	Radiometer	SEVIRI	Spinning enhanced visible and infrared imager

208	EUMETSAT	Imaging	VIRI	VIRI

multi-spectral
			radiometer (vis/IR)

220	ESA/	Spectrometer	GOME-2	Global ozone monitoring experiment-2
		EUMETSAT

221	CNES/	Atmospheric 	IASI	Infrared atmospheric sounding interferometer
		EUMETSAT	temperature and

humidity sounder

240	CAST	Communications	DCP	Data-collection platform transponder

245	CAST	Radiometer	CCD	High-resolution CCD camera

246	INPE	Atmospheric 	HSB	Humidity sounder/Brazil

temperature and

humidity sounder

248	INPE	Imaging multi-	OBA	Observador Brasileiro da Amazonia

spectral radiometer

(vis/IR)

250	CAST	Radiometer	WFI	Wide field imager

255	CAST	Spectrometer	IRMSS	Infrared multispectral scanner

260	ISRO	Precision orbit	BSS & FSS 
				transponders

261	ISRO	Precision orbit	DRT-S&R

262	ISRO	Communications	INSAT Comms	Communications package for INSAT
	268	ISRO	High-resolution 	HR-PAN	High-resolution panchromatic camera

optical imager

269	ISRO	Imaging	MSMR	Multifrequency scanning

multi-spectral	 	microwave radiometer

radiometer
			(passive microwave)

270	ISRO	Imaging	VHRR	Very high-resolution radiometer

multi-spectral
			radiometer (vis/IR)

271	ISRO	Imaging	WiFS	Wide field sensor

multi-spectral
			radiometer (vis/IR)

275	ISRO	High-resolution 	AWiFS	Advanced wide field sensor

optical imager

276	ISRO	High-resolution 	LISS-I	Linear imaging self scanner-I

optical imager

277	ISRO	High-resolution 	LISS-II	Linear imaging self scanner-II

optical imager

278	ISRO	High-resolution 	LISS-III	Linear imaging self scanner-III

optical imager

279	ISRO	High-resolution 	LISS-IV	Linear imaging self scanner-IV

optical imager

284	ISRO	High-resolution 	PAN	Panchromatic sensor

optical imager

Code	Agency	Type	Instrument short name	Instrument long name

285	ISRO	Imaging	MOS	Modular opto-electronic scanner

multi-spectral
			radiometer (vis/IR)

286	ISRO	Ocean colour 	OCM	Ocean colour monitor

Instrument

287	ASI		ROSA	Radio Occultation Sounder of the Atmosphere
	288	ISRO	Scatterometer	SCAT	Scatterometer

290	JMA	Communications	MTSAT Comms	Communications package for MTSAT

291	JMA	Communications	Himawari Comms	Communications package for Himawari

294	JMA	Imaging multi-	JAMI	Japanese Advanced Meteorological Imager

spectral radiometer

295	JMA	Imaging multi-	IMAGER/MTSAT-2	Imager/MTSAT-2

spectral radiometer

296	JMA	Imaging multi-	VISSR	Visible and infrared spin scan radiometer

spectral radiometer

297	JMA	Imaging multi-	AHI	Advanced Himawari Imager

spectral radiometer

300	NASA	Lidar	GLAS	Geoscience laser altimeter system

301	NASA	Precision orbit	LRA	Laser retroreflector array

302	NASA	Lidar	MBLA	Multi-beam laser altimeter

303	NASA	Lidar	CALIOP	Cloud-aerosol lidar with
					orthogonal polarization

309	NASA	Cloud profile and	CPR (Cloudsat)	Cloud profiling radar

rain radar

312	NASA	Radar	NSCAT	NASA scatterometer

313	NASA	Radar	SeaWinds	ADEOS II – NASA scatterometer

330	NASA	Earth radiation 	ACRIM	Active cavity radiometer irradiance monitor

budget radiometer

334	NASA	Total and profile	BUV	Backscatter ultraviolet instrument
			ozone

336	NASA	High-resolution 	ALI	Advanced land imager

optical imager

347	NASA	High-resolution 	ASTER	Advanced spaceborne thermal emission and

optical imager		reflection radiometer

348	NASA	Earth radiation 	CERES-2	Cloud and the Earth's radiant energy system

budget radiometer

351	NASA	Atmospheric 	GPSDR	GPS demonstration receiver

temperature and

humidity sounder

353	NASA	Total and profile	HiRDLS	High-resolution dynamics limb sounder

ozone

354	NASA	Total and profile	HRDI	High-resolution Doppler imager
			ozone

356	NASA	Radiometer	LIS	Lightning imaging sensor

358	NASA	Magnetic field,	PEM	Particle environment monitor

auroal imagery

scintillation boundary

359	NASA	Ocean colour 	SeaWiFS	Sea-viewing wide field-of-view sensor

instrument

Code	Agency	Type	Instrument short name	Instrument long name

360	NASA	Earth radiation 	SUSIM (UARS)	Solar ultraviolet irradiance monitor

budget radiometer

363	NASA	Total and profile	SBUV/1	Solar backscatter ultraviolet 1 instrument

ozone

365	NASA	Imaging multi-	TMI	TRMM microwave imager

spectral radiometer

(passive microwave)

366	NASA	Imaging multi-	JMR	JASON-1 microwave radiometer

spectral radiometer

(passive microwave)

367	NASA	Imaging multi-	AMR	Advanced microwave radiometer
			spectral radiometer
	369	NASA	Total and profile	LIMS	Limb infrared monitor of the stratosphere
			ozone

370	NASA	Total and profile	LRIR	Limb radiance inversion radiometer instrument
			ozone

371	NASA	Total and profile	EPIC	Earth polychromatic imaging camera
			ozone

372	NASA	Earth radiation 	NISTAR	NIST advanced radiometer

budget radiometer

373	NASA	Magnetic field,	Plasma-Mag

auroal imagery
			scintillation
			boundary

374	NASA	Other	XPS	XUV photometer system

375	NASA	Imaging multi-	VIRS	Visible infrared scanner

spectral radiometer

(vis/IR)

376	CNES	Multiple direction/	POLDER II	Polarization and directionality of the Earth's

polarization		reflectance-II
			radiometer

377	NASA	Earth radiation 	TIM	Total irradiance monitor

budget radiometer

379	NASA	Imaging multi-	WFC	Wide field camera

spectral radiometer

(vis/IR)

382	NASA	Spectro-radiometer	CLAES	Cryogenic limb array etalon spectrometer

383	NASA	Spectro-radiometer	HALOE	Halogen occultation experiment

384	NASA	Spectro-radiometer	ISAMS	Improved stratospheric and mesospheric

sounder

385	NASA	Spectro-radiometer	MISR	Multi-angle imaging spectroradiometer

386	NASA	Spectro-radiometer	MLS	Microwave limb sounder

387	NASA	Spectro-radiometer	MLS (EOS-Aura)	Microwave limb sounder (EOS-Aura)

389	NASA	Spectro-radiometer	MODIS	Moderate-resolution imaging 
					spectroradiometer

393	NASA	Gravity	HAIRS	High accuracy inter-satellite ranging system

394	NASA	Total and profile	OMI	Ozone measuring instrument
			ozone

395	NASA	Radiometer	Atmospheric corrector	Atmospheric corrector

396	NASA	Radiometer	Hyperion	Hyperspectral imager

Code	Agency	Type	Instrument short name	Instrument long name

399	NASA	Spectro-radiometer	SAGE I	Stratospheric aerosol and gas

experiment-I

400	NASA	Spectro-radiometer	SAGE II	Stratospheric aerosol and gas

experiment-II

401	NASA	Spectro-radiometer	SAGE III	Stratospheric aerosol and gas

experiment-III

402	NASA	Spectro-radiometer	SAMS	Stratospheric and mesospheric sounder

403	NASA	Spectro-radiometer	SAM-II	Stratospheric aerosol measurement-II

404	NASA	Spectro-radiometer	IRIS	Infrared interferometer spectrometer

405	NASA	Atmospheric 	GIFTS	Geosynchronous imaging Fourier transform

temperature and 		spectrometer

Humidity sounder

420	NASA	Spectrometer	AIRS	Atmospheric Infrared sounder

426	NASA	Spectrometer	SOLSTICE	Solar stellar irradiance comparison 
					experiment

430	NASA	Spectrometer	TES	Tropospheric emission spectrometer

431	NASA	Spectrometer	TOMS	Total ozone mapping spectrometer

450	JAXA	Communications	ADEOS Comms	Communications package for ADEOS

451	JAXA	Communications	DCS (JAXA)	Data-collection system (JAXA)

453	NASDA	Communications	GMS Comms	Communications package on GMS

454	NASDA	Communications	JERS-1 Comms	Communications package for JERS-1
	460	NASDA	Lidar	RIS	Retroreflector in space

461	NASDA	Radar	PR	Precipitation radar

462	NASDA	Imaging microwave 	SAR	Synthetic aperture radar

radar

470	JAXA	Imaging microwave 	PALSAR	Phased array type L-band synthetic aperture

radar		radar

478	JAXA	Imaging multi-	AMSR2	Advanced Microwave Scanning
			spectral radiometer 		Radiometer 2
			(passive microwave)

479	JAXA	Imaging multi-	AMSR-E	Advanced microwave scanning radiometer –

spectral radiometer 		EOS

(passive microwave)

480	JAXA	High-resolution 	PRISM (ALOS)	Panchromatic remote-sensing instrument for

optical imager		stereo mapping

481	JAXA	Radiometer	AMSR	Advanced microwave scanning 	radiometer

482	NASDA	High-resolution 	AVNIR	Advanced visible and near infrared radiometer

optical imager

483	JAXA	High-resolution 	AVNIR-2	Advanced visible and near infrared radiometer

optical imager		type 2

484	JAXA	Imager	GLI	Global imager

485	NASDA	Radiometer	MESSR	Multispectral electronic self scanning

radiometer

486	NASDA	Radiometer	MSR	Microwave scanning radiometer

487	NASDA	Radiometer	OCTS	Ocean colour and temperature scanner

488	NASDA	Radiometer	OPS	Optical sensor

489	NASDA	Radiometer	VISSR (GMS-5)	Visible and infrared spin scan radiometer

(GMS-5)

490	NASDA	Radiometer	VTIR	Visible and thermal infrared radiometer

Code	Agency	Type	Instrument short name	Instrument long name

510	NASDA	Spectrometer	ILAS-I	Improved limb atmospheric spectrometer

511	NASDA	Spectrometer	ILAS-II	Improved limb atmospheric spectrometer

512	NASDA	Spectrometer	IMG	Inferometric monitor of greenhouse gases

514	NASDA	Space environment	SEM	Space environment monitor (NASDA)

515	JAXA	Total and profile	SOFIS	Solar occultation Fourier transform

ozone		spectrometer for inclined orbit satellite

516	JAXA	Spectrometer	TANSO-FTS	Thermal and Near infrared Sensor for carbon

Observations (TANSO) Fourier Transform

Spectrometer (FTS)

517	JAXA	Imager	TANSO-CAI	Thermal and Near infrared Sensor for carbon

Observations (TANSO) Cloud and Aerosol

Imager (CAI)

518	JAXA	Cloud and precipi-	DPR	Dual-frequency precipitation radar
			tation radar

519	NASA	MW imaging/sound- 	GMI	GPM microwave imager
			ing radiometer, 
			conical scanning

540	NOAA	Communications	DCS (NOAA)	Data-collection system (NOAA)

541	NOAA	Communications	GOES Comms	Communications package on GOES

542	NOAA	Communications	LANDSAT Comms	Communications package for LANDSAT

543	NOAA	Communications	NOAA Comms	Communications package for NOAA

544	NOAA	Communications	S&R (GOES)	Search and rescue

545	NOAA	Communications	S&R (NOAA)	Search and rescue

546	NOAA	Communications	WEFAX	Weather facsimile

547	NOAA	Spectrometer	SEM (GOES)	Space environment monitor

550	NOAA	Magnetic field	SSM	Special sensor magnetometer

551	NOAA	Magnetic field	SSJ/4	Special sensor precipitating

plasma monitor

552	NOAA	Space environment	SSIES-2	Special sensor ionospheric plasma

drift/scintillation meter

553	NOAA	Space environment	SSB/X-2	Special sensor gamma ray particle detector

570	NOAA	Radiometer	AMSU-A	Advanced microwave sounding unit-A
	574	NOAA	Radiometer	AMSU-B	Advanced microwave sounding unit-B
	580	NOAA	Radiometer	ATOVS (HIRS/3 + 	Advanced TIROS operational vertical sounder

AMSU + AVHRR/3)

590	NOAA	Radiometer	AVHRR/2	Advanced very high-resolution radiometer/2

591	NOAA	Radiometer	AVHRR/3	Advanced very high-resolution radiometer/3

592	NOAA	Radiometer	AVHRR/4	Advanced very high-resolution radiometer/4

600	NOAA	Radiometer	ERBE	Earth's radiation budget experiment

601	NOAA	Radiometer	ETM+	Enhanced thematic mapper

605	NOAA	Radiometer	HIRS/2	High-resolution infrared sounder/2

606	NOAA	Radiometer	HIRS/3	High-resolution infrared sounder/3

607	NOAA	Radiometer	HIRS/4	High-resolution infrared sounder/4

615	NOAA	Radiometer	IMAGER	Imager

616	NOAA	Imaging multi-	VIIRS	Visible/infrared imager radiometer suite

spectral radiometer

(vis/IR)

620	NOAA	Atmospheric 	CrIRS/NP	Cross-track infrared sounder/NPOESS

temperature and

humidity sounder

Code	Agency	Type	Instrument short name	Instrument long name

621	NOAA	Atmospheric 	ATMS	Advanced technology microwave sounder

temperature and

humidity sounder

622	NOAA	Radiometer	MSS	Multispectral scanning system

623	NOAA	Radiometer	MSU	Microwave sounding unit

624	NOAA	Radiometer	SBUV/2	Solar backscatter ultraviolet instrument/2

625	NOAA	Radiometer	SBUV/3	Solar backscatter ultraviolet instrument/3

626	NOAA	Radiometer	SOUNDER	SOUNDER

627	NOAA	Radiometer	SSU	Stratospheric sounding unit

628	NOAA	Radiometer	TM	Thematic mapper

629	NOAA	Radiometer	TOVS (HIRS/2 + 	TIROS operational vertical sounder

MSU + SSU)

630	NOAA	Radiometer	VAS	VISSR atmospheric sounder

631	NOAA	Radiometer	SSZ

645	NOAA	Spectrometer	SEM	Space environment monitor

650	NRSCC	Radiometer	MVIRSR (10 channel)	Multispectral visible and infrared

scan radiometer

651	NRSCC	Radiometer	MVIRSR (3 channel)	Multispectral visible and infrared

scan radiometer

652	NRSCC	Radiometer	MVIRSR (5 channel)	Multispectral visible and infrared

scan radiometer

670	NSAU	Radar	RLSBO	Side looking microwave radar

680	NSAU	High-resolution 	MSU-EU	Multispectral radiometer with high resolution

optical imager

681	NSAU	Imaging multi-	MSU-UM	Visible multispectral radiometer

spectral radio-

meter (vis/IR)

682	NSAU	Radiometer	RM-08	Imaging microwave radiometer

683	NSAU	High-resolution 	SU-UMS	Stereo radiometer with high resolution

optical imager

684	NSAU	High-resolution 	SU-VR	Visible radiometer with high resolution

optical imager

685	NSAU	Radiometer	TRASSER

700	ROSCOSMOS	Communications	KONDOR-2	Data-collection and transmission system
	701	ROSCOSMOS	Communications	BRK

710	ROSCOSMOS	Lidar	ALISSA	Backscatter lidar

712	ROSCOSMOS	Lidar	Balkan-2 lidar

715	ROSCOSMOS	Lidar	MK-4

716	ROSCOSMOS	Lidar	MK-4M

730	ROSCOSMOS	Radar	Greben	Radar altimeter

731	ROSCOSMOS	Radar	SAR-10	Synthetic aperture radar

732	ROSCOSMOS	Radar	SAR-3	Synthetic aperture radar

733	ROSCOSMOS	Radar	SAR-70	Synthetic aperture radar

740	ROSCOSMOS	Radar	SLR-3	Side looking radar

745	ROSCOSMOS	Radar	Travers SAR

750	ROSCOSMOS	Radiometer	174-K	Temperature and humidity profiler

751	ROSCOSMOS	Radiometer	BTVK	Scanning television radiometer

752	ROSCOSMOS	Radiometer	Chaika	Scanning infrared radiometer

753	ROSCOSMOS	Radiometer	DELTA-2	Multispectral microwave scanner

755	ROSCOSMOS	Radiometer	IKAR-D	Multispectral microwave scanner

756	ROSCOSMOS	Radiometer	IKAR-N	Multispectral microwave scanner

Code	Agency	Type	Instrument short name	Instrument long name

757	ROSCOSMOS	Radiometer	IKAR-P	Multispectral microwave scanner

760	ROSCOSMOS	Radiometer	ISP

761	ROSCOSMOS	Radiometer	KFA-1000	Photographic camera

762	ROSCOSMOS	Radiometer	KFA-200	Photographic camera

763	ROSCOSMOS	Radiometer	KFA-3000	Photographic camera

770	ROSCOSMOS	Radiometer	Klimat	Scanning infrared radiometer

771	ROSCOSMOS	Radiometer	Klimat-2	Scanning infrared radiometer

775	ROSCOSMOS	Radiometer	MIRAS

776	ROSCOSMOS	Radiometer	MIVZA

777	ROSCOSMOS	Radiometer	MIVZA-M	Microwave scanning radiometer

780	ROSCOSMOS	Radiometer	MR-2000

781	ROSCOSMOS	Radiometer	MR-2000M

785	ROSCOSMOS	Radiometer	MR-900	Scanning telephotometer

786	ROSCOSMOS	Radiometer	MR-900B	Scanning visual band telephotometer
	790	ROSCOSMOS	Radiometer	MSU-E	Multispectral high-resolution

electronic scanner

791	ROSCOSMOS	Radiometer	MSU-E1	Multispectral high-resolution

electronic scanner

792	ROSCOSMOS	Radiometer	MSU-E2	Multispectral high-resolution

electronic scanner

793	ROSCOSMOS	Radiometer	MSU-M

794	ROSCOSMOS	Radiometer	MSU-S	Multispectral medium-resolution scanner

795	ROSCOSMOS	Radiometer	MSU-SK	Multispectral medium-resolution 
					conical scanner

796	ROSCOSMOS	Radiometer	MSU-V	Multispectral high-resolution 
					conical scanner

810	ROSCOSMOS	Radiometer	MTZA	Scanning microwave radiometer

815	ROSCOSMOS	Imaging multi-	MZOAS	Scanning microwave radiometer

spectral radiometer

(passive microwave)

820	ROSCOSMOS	Imaging multi-	R-225	Single channel microwave radiometer

spectral radiometer

(passive microwave)

821	ROSCOSMOS	Radiometer	R-400

822	ROSCOSMOS	Radiometer	R-600	Single channel microwave radiometer

830	ROSCOSMOS	Radiometer	RMS	Radiation measurement system

835	ROSCOSMOS	Radiometer	TV camera

836	ROSCOSMOS	Radiometer	SILVA

840	ROSCOSMOS	Spectro-radiometer	SROSMO	Spectroradiometer for ocean monitoring
	850	ROSCOSMOS	Spectrometer	BUFS-2	Backscatter spectrometer/2

851	ROSCOSMOS	Spectrometer	BUFS-4	Backscatter spectrometer/4

855	ROSCOSMOS	Spectrometer	ISTOK-1	Infrared spectrometer

856	ROSCOSMOS	Spectrometer	SFM-2	Spectrometer to measure direct

solar radiation

857	ROSCOSMOS	Spectrometer	DOPI

858	ROSCOSMOS	Spectrometer	KGI-4

859	ROSCOSMOS	Spectrometer	Ozon-M

860	ROSCOSMOS	Spectrometer	RMK-2

900	NOAA	Radiometer	MAXIE	Magnetospheric atmospheric X-ray imaging

experiment

Code	Agency	Type	Instrument short name	Instrument long name

901	NOAA	Radiometer	OLS	Operational linescan system

905	NOAA	Radiometer	SSM/I	Mission sensor microwave imager

906	NOAA	Radiometer	SSM/T-1	Mission sensor microwave

temperature sounder

907	NOAA	Radiometer	SSM/T-2	Mission sensor microwave water

vapour sounder

908	NOAA	Radiometer	SSMIS	Special sensor microwave imager sounder

910	NOAA	Radiometer	SXI	Solar X-ray imager

930	NOAA	Spectrometer	EHIC	Energetic heavy ion composition experiment
	931	NOAA	Spectrometer	X-ray astronomy 
				payload

932	NRSCC	Imaging multi-	IVISSR (FY-2)	Improved multispectral visible and Infrared

spectral radiometer 		scan radiometer (5 channels)

(vis/IR)

933	NRSCC	Atmospheric 	IRAS	Infrared atmospheric sounder

temperature and

humidity sounder

934	NRSCC	Atmospheric 	MWAS	Microwave atmospheric sounder

temperature and

humidity sounder

935	NRSCC	Atmospheric 	IMWAS	Improved Microwave atmospheric sounder

temperature and

humidity sounder

936	NRSCC	Atmospheric 	MWHS	Microwave humidity sounder

temperature and

humidity sounder

937	NRSCC	Imaging multi-	MVIRS	Moderate resolution visible and

spectral radiometer 		infrared imaging spectroradiometer

(vis/IR)

938	NRSCC	Imaging multi-	MWRI	Microwave radiation imager

spectral radiometer

(passive microwave)

940	ROSCOSMOS	Atmospheric 	MTVZA-OK	Scanning microwave radiometer

temperature and

humidity sounder

941	CNES	Atmospheric 	SAPHIR

temperature and

humidity sounder

942	CNES	Microwave imager	MADRAS	Microwave Analysis and Detection 
					of Rain and Atmospheric Structures

944	NOAA	Radar altimeter	ALT	Altimeter

945	NOAA	Earth radiation 	TSIS	Total solar irradiance sensor

budget radiometer

946	NOAA	Imaging multi-	CMIS	Conical-scanning microwave

spectral radiometer 		imager/sounder

(passive microwave)

947	NOAA	Total and profile	OMPS	Ozone mapping and profiler suite				ozone

948	NOAA	Space environment 	GPSOS	Global positioning system occultation sensor

atmospheric

temperature and

humidity sounder

Code	Agency	Type	Instrument short name	Instrument long name

949	NOAA	Magnetic field,	SESS	Space environmental sensor suite

auroal imagery

scintillation boundary

950	NRSCC	Imaging multi-	VIRR	Multispectral visible and infrared scan

spectral radiometer 		radiometer (10 channels)

(vis/IR)

951	NRSCC	Total and profile	TOM	Total ozone mapper
			ozone

952	NRSCC	Total and profile	OP	Ozone profiler
			ozone

953	CMA	Microwave sound-	MWHS-2	Microwave humidity sounder-2

ing radiometer, 
			crosstrack scanning

954	CMA	Microwave sound-	MWTS-2	Microwave temperature sounder-2
			ing radiometer, 
			crosstrack scanning

955	CMA	Cross-nadir scan-	HIRAS	Hyperspectral infrared atmospheric sounder
			ning IR sounder

956–999	Reserved

1000–2046	Reserved for long-term future use

2047		Missing value

COMMON CODE TABLE C–11:	Originating/generating centres

BUFR 0 01 035

Common Code table	CREX Edition 2, ooooo in Group Poooooppp in Section 1

GRIB Edition 2, Octets 6–7 in Section 1

BUFR Edition 4, Octets 5–6 in Section 1

CREX Edition 2	GRIB Edition 2

B 01 035	Octets 6–7 in Section 1

(5 characters)	BUFR Edition 4

and Group 3	0 01 035 (16 bits)

in Section 1	and Octets 5–6 in Section 1

00000	0	WMO Secretariat

00001–00009: WMCs

00001	1	Melbourne

00002	2	Melbourne

00003	3	)

00004	4	Moscow

00005	5	Moscow

00006	6	)

00007	7	US National Weather Service, National Centres for

Environmental Prediction (NCEP)

00008	8	US National Weather Service Telecommunications

Gateway (NWSTG)

00009	9	US National Weather Service – Other

00010–00025: Centres in Region I

00010	10	Cairo (RSMC)

00011	11	)

00012	12	Dakar (RSMC)

00013	13	)

00014	14	Nairobi (RSMC)

00015	15	)

00016	16	Casablanca (RSMC)

00017	17	Tunis (RSMC)

00018	18	Tunis–Casablanca (RSMC)

00019	19	)

00020	20	Las Palmas

00021	21	Algiers (RSMC)

00022	22	ACMAD

00023	23	Mozambique (NMC)

00024	24	Pretoria (RSMC)

00025	25	La Réunion (RSMC)

00026–00040: Centres in Region II

00026	26	Khabarovsk (RSMC)

00027	27	)

00028	28	New Delhi (RSMC)

00029	29	)

CREX Edition 2	GRIB Edition 2

B 01 035	Octets 6–7 in Section 1

(5 characters)	BUFR Edition 4

and Group 3	0 01 035 (16 bits)

in Section 1	and Octets 5–6 in Section 1

00030	30	Novosibirsk (RSMC)

00031	31	)

00032	32	Tashkent (RSMC)

00033	33	Jeddah (RSMC)

00034	34	Tokyo (RSMC), Japan Meteorological Agency

00035	35	)

00036	36	Bangkok

00037	37	Ulaanbaatar

00038	38	Beijing (RSMC)

00039	39	)

00040	40	Seoul

00041–00050: Centres in Region III

00041	41	Buenos Aires (RSMC)

00042	42	)

00043	43	Brasilia (RSMC)

00044	44	)

00045	45	Santiago

00046	46	Brazilian Space Agency ­ INPE

00047	47	Colombia (NMC)

00048	48	Ecuador (NMC)

00049	49	Peru (NMC)

00050	50	Venezuela (Bolivarian Republic of) (NMC)

00051–00063: Centres in Region IV

00051	51	Miami (RSMC)

00052	52	Miami RSMC, National Hurricane Centre

00053	53	Montreal (RSMC)

00054	54	)

00055	55	San Francisco

00056	56	ARINC Centre

00057	57	US Air Force – Air Force Global Weather Central

00058	58	Fleet Numerical Meteorology and Oceanography 
			Center, Monterey, CA, United States

00059	59	The NOAA Forecast Systems Laboratory, Boulder, CO,
			United States

00060	60	United States National Center for Atmospheric Research
			(NCAR)

00061	61	Service ARGOS – Landover

00062	62	US Naval Oceanographic Office

00063	63	International Research Institute for Climate and Society
			(IRI)

CREX Edition 2	GRIB Edition 2

B 01 035	Octets 6–7 in Section 1

(5 characters)	BUFR Edition 4

and Group 3	0 01 035 (16 bits)

in Section 1	and Octets 5–6 in Section 1

00064–00073: Centres in Region V

00064	64	Honolulu (RSMC)

00065	65	Darwin (RSMC)

00066	66	)

00067	67	Melbourne (RSMC)

00068	68	Reserved

00069	69	Wellington (RSMC)

00070	70	)

00071	71	Nadi (RSMC)

00072	72	Singapore

00073	73	Malaysia (NMC)

00074–00099: Centres in Region VI

00074	74	UK Meteorological Office – Exeter (RSMC)

00075	75	)

00076	76	Moscow (RSMC)

00077	77	Reserved

00078	78	Offenbach (RSMC)

00079	79	)

00080	80	Rome (RSMC)

00081	81	)

00082	82	Norrköping

00083	83	)

00084	84	Toulouse (RSMC)

00085	85	Toulouse (RSMC)

00086	86	Helsinki

00087	87	Belgrade

00088	88	Oslo

00089	89	Prague

00090	90	Episkopi

00091	91	Ankara

00092	92	Frankfurt/Main

00093	93	London (WAFC)

00094	94	Copenhagen

00095	95	Rota

00096	96	Athens

00097	97	European Space Agency (ESA)

00098	98	European Centre for Medium Range Weather Forecasts
			(ECMWF) (RSMC)

00099	99	De Bilt

CREX Edition 2	GRIB Edition 2

B 01 035	Octets 6–7 in Section 1

(5 characters)	BUFR Edition 4

and Group 3	0 01 035 (16 bits)

in Section 1	and Octets 5–6 in Section 1

Additional Centres

00100	100	Brazzaville

00101	101	Abidjan

00102	102	Libya (NMC)

00103	103	Madagascar (NMC)

00104	104	Mauritius (NMC)

00105	105	Niger (NMC)

00106	106	Seychelles (NMC)

00107	107	Uganda (NMC)

00108	108	United Republic of Tanzania (NMC)

00109	109	Zimbabwe (NMC)

00110	110	Hong Kong, China

00111	111	Afghanistan (NMC)

00112	112	Bahrain (NMC)

00113	113	Bangladesh (NMC)

00114	114	Bhutan (NMC)

00115	115	Cambodia (NMC)

00116	116	Democratic People's Republic of Korea (NMC)

00117	117	Islamic Republic of Iran (NMC)

00118	118	Iraq (NMC)

00119	119	Kazakhstan (NMC)

00120	120	Kuwait (NMC)

00121	121	Kyrgyzstan (NMC)

00122	122	Lao People's Democratic Republic (NMC)

00123	123	Macao, China

00124	124	Maldives (NMC)

00125	125	Myanmar (NMC)

00126	126	Nepal (NMC)

00127	127	Oman (NMC)

00128	128	Pakistan (NMC)

00129	129	Qatar (NMC)

00130	130	Yemen (NMC)

00131	131	Sri Lanka (NMC)

00132	132	Tajikistan (NMC)

00133	133	Turkmenistan (NMC)

00134	134	United Arab Emirates (NMC)

00135	135	Uzbekistan (NMC)

00136	136	Viet Nam (NMC)

00137–00139	137–139	Reserved for other centres

00140	140	Bolivia (Plurinational State of) (NMC)

CREX Edition 2	GRIB Edition 2

B 01 035	Octets 6–7 in Section 1

(5 characters)	BUFR Edition 4

and Group 3	0 01 035 (16 bits)

in Section 1	and Octets 5–6 in Section 1

00141	141	Guyana (NMC)

00142	142	Paraguay (NMC)

00143	143	Suriname (NMC)

00144	144	Uruguay (NMC)

00145	145	French Guiana

00146	146	Brazilian Navy Hydrographic Centre

00147	147	National Commission on Space Activities (CONAE) –

Argentina

00148–00149	148–149	Reserved for other centres

00150	150	Antigua and Barbuda (NMC)

00151	151	Bahamas (NMC)

00152	152	Barbados (NMC)

00153	153	Belize (NMC)

00154	154	British Caribbean Territories Centre

00155	155	San José

00156	156	Cuba (NMC)

00157	157	Dominica (NMC)

00158	158	Dominican Republic (NMC)

00159	159	El Salvador (NMC)

00160	160	US NOAA/NESDIS

00161	161	US NOAA Office of Oceanic and Atmospheric Research

00162	162	Guatemala (NMC)

00163	163	Haiti (NMC)

00164	164	Honduras (NMC)

00165	165	Jamaica (NMC)

00166	166	Mexico

00167	167	Curaçao and Sint Maarten (NMC)

00168	168	Nicaragua (NMC)

00169	169	Panama (NMC)

00170	170	Saint Lucia (NMC)

00171	171	Trinidad and Tobago (NMC)

00172	172	French Departments in RA IV

00173	173	US National Aeronautics and Space Administration (NASA)
	00174	174	Integrated Science Data Management/Marine

Environmental Data Service (ISDM/MEDS – Canada)

00175	175	University Corporation for Atmospheric Research (UCAR) 
			– United States

00176	176	Cooperative Institute for Meteorological Satellite 
			Studies (CIMSS) – United States

00177	177	NOAA National Ocean Service – United States

00178–00189	178–189	Reserved for other centres

CREX Edition 2	GRIB Edition 2

B 01 035	Octets 6–7 in Section 1

(5 characters)	BUFR Edition 4

and Group 3	0 01 035 (16 bits)

in Section 1	and Octets 5–6 in Section 1

00190	190	Cook Islands (NMC)

00191	191	French Polynesia (NMC)

00192	192	Tonga (NMC)

00193	193	Vanuatu (NMC)

00194	194	Brunei Darussalam (NMC)

00195	195	Indonesia (NMC)

00196	196	Kiribati (NMC)

00197	197	Federated States of Micronesia (NMC)

00198	198	New Caledonia (NMC)

00199	199	Niue

00200	200	Papua New Guinea (NMC)

00201	201	Philippines (NMC)

00202	202	Samoa (NMC)

00203	203	Solomon Islands (NMC)

00204	204	National Institute of Water and Atmospheric Research
			(NIWA – New Zealand)

00205–00209	205–209	Reserved for other centres

00210	210	Frascati (ESA/ESRIN)

00211	211	Lannion

00212	212	Lisboa

00213	213	Reykjavik

00214	214	Madrid

00215	215	Zurich

00216	216	Service ARGOS Toulouse

00217	217	Bratislava

00218	218	Budapest

00219	219	Ljubljana

00220	220	Warsaw

00221	221	Zagreb

00222	222	Albania (NMC)

00223	223	Armenia (NMC)

00224	224	Austria (NMC)

00225	225	Azerbaijan (NMC)

00226	226	Belarus (NMC)

00227	227	Belgium (NMC)

00228	228	Bosnia and Herzegovina (NMC)

00229	229	Bulgaria (NMC)

00230	230	Cyprus (NMC)

00231	231	Estonia (NMC)

00232	232	Georgia (NMC)

CREX Edition 2	GRIB Edition 2

B 01 035	Octets 6–7 in Section 1

(5 characters)	BUFR Edition 4

and Group 3	0 01 035 (16 bits)

in Section 1	and Octets 5–6 in Section 1

00233	233	Dublin

00234	234	Israel (NMC)

00235	235	Jordan (NMC)

00236	236	Latvia (NMC)

00237	237	Lebanon (NMC)

00238	238	Lithuania (NMC)

00239	239	Luxembourg

00240	240	Malta (NMC)

00241	241	Monaco

00242	242	Romania (NMC)

00243	243	Syrian Arab Republic (NMC)

00244	244	The former Yugoslav Republic of Macedonia (NMC)

00245	245	Ukraine (NMC)

00246	246	Republic of Moldova (NMC)

00247	247	Operational Programme for the Exchange of weather
			RAdar information (OPERA) – EUMETNET

00248	248	Montenegro (NMC)

00249	249	Barcelona Dust Forecast Center

00250	250	COnsortium for Small scale MOdelling (COSMO)

00251	251	Meteorological Cooperation on Operational NWP
			(MetCoOp)

00252	252	Max Planck Institute for Meteorology (MPI-M)

00253	253	Reserved for other centres

00254	254	EUMETSAT Operation Centre

00255	255	Not to be used

00256	256	Angola (NMC)

00257	257	Benin (NMC)

00258	258	Botswana (NMC)

00259	259	Burkina Faso (NMC)

00260	260	Burundi (NMC)

00261	261	Cameroon (NMC)

00262	262	Cabo Verde (NMC)

00263	263	Central African Republic (NMC)

00264	264	Chad (NMC)

00265	265	Comoros (NMC)

00266	266	Democratic Republic of the Congo (NMC)

00267	267	Djibouti (NMC)

00268	268	Eritrea (NMC)

00269	269	Ethiopia (NMC)

CREX Edition 2	GRIB Edition 2

B 01 035	Octets 6–7 in Section 1

(5 characters)	BUFR Edition 4

and Group 3	0 01 035 (16 bits)

in Section 1	and Octets 5–6 in Section 1

00270	270	Gabon (NMC)

00271	271	Gambia (NMC)

00272	272	Ghana (NMC)

00273	273	Guinea (NMC)

00274	274	Guinea-Bissau (NMC)

00275	275	Lesotho (NMC)

00276	276	Liberia (NMC)

00277	277	Malawi (NMC)

00278	278	Mali (NMC)

00279	279	Mauritania (NMC)

00280	280	Namibia (NMC)

00281	281	Nigeria (NMC)

00282	282	Rwanda (NMC)

00283	283	Sao Tome and Principe (NMC)

00284	284	Sierra Leone (NMC)

00285	285	Somalia (NMC)

00286	286	Sudan (NMC)

00287	287	Swaziland (NMC)

00288	288	Togo (NMC)

00289	289	Zambia (NMC)

00290–65534	290–65534	Reserved for other centres

65535	65535	Missing value

65536–99999	Not applicable	Not used

Notes:

(1)	The closed bracket sign ")" indicates that the corresponding code figure is reserved for the previously named centre.

(2)	With GRIB or BUFR, to indicate whether the originating/generating centre is a sub-centre or not, the following procedure should be applied:

In GRIB edition 2, use octets 8–9 of section 1, or in BUFR edition 4, use octets 7–8 of section 1, with the following meaning:

Code figure

0	Not a sub-centre, the originating/generating centre is the centre defined by octets 6–7 in section 1 of GRIB edition 2, or by octets 5–6 in section 1 of BUFR edition 4.

1 to 254	Identifier of the sub-centre which is the originating/generating centre. The identifier of the sub-centre is allocated by the associated centre, which is defined by octets 6–7 in section 1 of GRIB edition 2 or by octets 5–6 in section 1 of BUFR edition 4. The sub-centre identifiers should be supplied to the WMO Secretariat by the associated centre(s) for publication.

(3)	For the definitions of sub-centres provided to the WMO Secretariat, see Common Code table C–12.

COMMON CODE TABLE C–12:	Sub-centres of originating centres defined by entries in
Common Code tables C–1 or C–11

ORIGINATING CENTRES	SUB-CENTRES

C–1, C–11 or C–12	BUFR 0 01 034

BUFR Edition 3, Octet 5 in Section 1

BUFR Edition 4, Octets 7–8 in Section 1

GRIB Edition 1, Octet 26 in Section 1

GRIB Edition 2, Octets 8–9 in Section 1

CREX Edition 2, ppp in Group Poooooppp
	in Section 1

Code figure	Name	Code figure	Name

0	No sub-centre

REGION II

34	Tokyo (RSMC), Japan 	207	Syowa

Meteorological Agency	240	Kiyose

241	Reanalysis project

39	Beijing (RSMC)	225	Beijing

226	Guangzhou

228	Urumuqi

40	Seoul	243	Seoul

245	Jincheon

110	Hong-Kong, China	229	Hong-Kong

REGION III

46	Brazilian Space Agency ­ 	10	Cachoeira Paulista (INPE)

INPE	11	Cuiaba (INPE)

12	Brasilia (INMET)

13	Fortaleza (FUNCEME)

14	Natal (Navy Hygrog. Centre)

15	Manaus (SIVAM)

16	Natal (INPE)

17	Boa Vista

25	São Paulo University - USP

147	National Commission on 	10	Córdoba

Space Activities (CONAE) – 	15	Ushuaia

Argentina	20	Marambio

30	Santiago de Chile

40	Punta Arenas

50	Base Presidente Frei

60	Cotopaxi

REGION IV

7	US National Weather Service,	1	NCEP Reanalysis Project

NCEP	2	NCEP Ensemble Products

3	NCEP Central Operations

4	Environmental Modeling Center

5	Weather Prediction Center

6	Ocean Prediction Center

7	Climate Prediction Center

8	Aviation Weather Center

9	Storm Prediction Center

10	National Hurricane Center

11	NWS Techniques Development Laboratory

12	NESDIS Office of Research and Applications

13	Federal Aviation Administration

14	NWS Meteorological Development Laboratory

ORIGINATING CENTRES	SUB-CENTRES

C–1, C–11 or C–12	BUFR 0 01 034

BUFR Edition 3, Octet 5 in Section 1

BUFR Edition 4, Octets 7–8 in Section 1

GRIB Edition 1, Octet 26 in Section 1

GRIB Edition 2, Octets 8–9 in Section 1

CREX Edition 2, ppp in Group Poooooppp
	in Section 1

Code figure	Name	Code figure	Name

REGION IV (continued)

7	US National Weather Service,	15	North American Regional Reanalysis Project
		NCEP	16	Space Weather Prediction Center
			17	ESRL Global Systems Division

160	United States NOAA/NESDIS	1	National Climatic Data Center

2	National Geophysical Data Center

3	National Oceanographic Data Center

4	Center for Satellite Applications and Research
				(STAR)

5	Joint Polar Satellite System

10	Tromso (Norway)

11	McMurdo (Antarctica)

161	United States NOAA Office of 	1	Great Lakes Environmental Research Laboratory

Oceanic and Atmospheric	2	Earth System Research Laboratory

Research (NOAA/OAR)	3	Atlantic Oceanographic and Meteorological

Laboratory

4	Pacific Marine Environmental Laboratory
			5	Air Resources Laboratory
			6	Geophysical Fluid Dynamics Laboratory
			7	National Severe Storms Laboratory

173	United States National 	1	Ames Research Center

Aeronautics and Space	2	Dryden Flight Research Center

Administration (NASA)	3	Glenn Research Center

4	Goddard Space Flight Center

5	Jet Propulsion Laboratory

6	Johnson Space Center

7	Kennedy Space Center

8	Langley Research Center

9	Marshall Space Flight Center

10	Stennis Space Center

11	Goddard Institute for Space Studies

12	Independent Verification and Validation Facility

13	NASA Shared Service Center

14	Wallops Flight Facility

176	Cooperative Institute for 	10	Tromso (Norway)

Meteorological	11	McMurdo (Antarctica)

Satellite Studies (CIMSS) –	12	Sodankyla (Finland)

United States	13	Fairbanks (United States)

14	Barrow (United States)

15	Rothera (Antarctica)

177	NOAA National Ocean Service	1	Centre for Operational Oceanographic Products

– United States	and Services

2	Coastal Survey Development Laboratory

ORIGINATING CENTRES	SUB-CENTRES

C–1, C–11 or C–12	BUFR 0 01 034

BUFR Edition 3, Octet 5 in Section 1

BUFR Edition 4, Octets 7–8 in Section 1

GRIB Edition 1, Octet 26 in Section 1

GRIB Edition 2, Octets 8–9 in Section 1

CREX Edition 2, ppp in Group Poooooppp
	in Section 1

Code figure	Name	Code figure	Name

REGION V

2	Melbourne	201	Casey

203	Davis

210	Alice Springs

211	Melbourne Crib Point 1

214	Darwin

217	Perth

219	Townsville

232	Fiji

235	Noumea

237	Papeete

250	Vladivostock

251	Guam

252	Honolulu

69	Wellington (RSMC)	204	National Institute of Water and Atmospheric

Research (NIWA – New Zealand)

205	Niue

206	Rarotonga (Cook Islands)

207	Apia (Samoa)

208	Tonga

209	Tuvalu

210	Kiribati

211	Tokelau

243	Kelburn

72	Singapore	249	Singapore

191	French Polynesia (NMC)	1	RARS station of Tahiti (French Polynesia)

204	National Institute of Water 	101	Maupia

and Atmospheric Research 	102	Lauder

(NIWA – New Zealand)

REGION VI

74	UK Met Office, Exeter (RSMC)	1	Shanwick Oceanic Area Control Centre

2	Fucino

3	Gatineau

4	Maspalomas (Spain)

5	ESA ERS Central Facility

6	Prince Albert

7	West Freugh

13	Tromso

21	Agenzia Spaziale Italiana (Italy)

22	Centre National de la Recherche

Scientifique (France)

23	GeoForschungs Zentrum (Germany)

24	Geodetic Observatory Pecny

(Czech Republic)

ORIGINATING CENTRES	SUB-CENTRES

C–1, C–11 or C–12	BUFR 0 01 034

BUFR Edition 3, Octet 5 in Section 1

BUFR Edition 4, Octets 7–8 in Section 1

GRIB Edition 1, Octet 26 in Section 1

GRIB Edition 2, Octets 8–9 in Section 1

CREX Edition 2, ppp in Group Poooooppp
	in Section 1

Code figure	Name	Code figure	Name

REGION VI (continued)

74	UK Met Office, Exeter (RSMC)	25	Institut d'Estudis Espacials de Catalunya (Spain)

26	Federal Office of Topography (Switzerland)

27	Nordic Commission of Geodesy (Norway)

28	Nordic Commission of Geodesy (Sweden)

29	Institute Géographique National (France) –

Service de géodésie

30	Bundesamt für Kartographie und Geodäsie

(Germany)

31	Institute of Engineering Satellite Surveying 
				and Geodesy (United Kingdom)

32	Joint Operational Meteorology and

Oceanography Centre (JOMOC)

33	Koninklijk Nederlands Meteorologisch

Institut (Netherlands)

34	Nordic GPS Atmospheric Analysis centre (Sweden)

35	Instituto Geografico Nacional de España (Spain)

36	Met Éireann (Ireland)

37	Royal Observatory of Belgium (Belgium)

78	Offenbach (RSMC)	10	POLARA (Polarimetric Radar Algorithms instance)

64	Bundeswehr Geoinformation Office (BGIO)

110	NowCast mobile (Lightning data)

221	Schleswig-Holstein, Traffic Operations
				Computing Centre (TOCC)
				Kiel/Neumünster

78	Offenbach (RSMC)	222	Hamburg, TOCC Hamburg

(continued)	223	Niedersachsen, TOCC Hannover
			224	Austria (NMC)

225	Nordrhein-Westfalen, TOCC Kamen
				Leverkusen

226	Hessen, TOCC Rüsselsheim

227	Rheinland-Pfalz, TOCC Koblenz

228	Baden-Württemberg, TOCC Ludwigsburg

229	Bayern, TOCC Freimann

230	Saarland, TOCC Rohrbach

231	Bayern, Autobahn directorate Nordbayern

232	Brandenburg, TOCC Stolpe

233	Mecklenburg-Vorpommern, TOCC Malchow

234	Sachsen, TOCC Dresden

235	Sachsen-Anhalt, TOCC Halle

236	Thüringen, TOCC Erfurt

237	EasyWay – Meteotrans
			254	EUMETSAT

80	Rome (RSMC)	101	Albania (NMC)

85	Toulouse (RSMC)	200	Institut National de l'Environnement Industriel et

des Risques (France)

201	Rheinisches Institut für Umweltforschung an der

Universität zu Köln E.V. (Germany)

ORIGINATING CENTRES	SUB-CENTRES

C–1, C–11 or C–12	BUFR 0 01 034

BUFR Edition 3, Octet 5 in Section 1

BUFR Edition 4, Octets 7–8 in Section 1

GRIB Edition 1, Octet 26 in Section 1

GRIB Edition 2, Octets 8–9 in Section 1

CREX Edition 2, ppp in Group Poooooppp
	in Section 1

Code figure	Name	Code figure	Name

REGION VI (continued)

89	Prague (RTH)	1	Solar and Ozone Observatory Hradec Kralove

96	Athens	1	Cyprus (NMC)

227	Belgium (NMC)	1	Luxembourg (NMC)

250	COSMO (COnsortium for	76	Roshydromet (Russian Federation)

Small scale MOdelling)	78	Deutscher Wetterdienst (Germany)

80	Ufficio Generale Spazio Aereo e Meteorologia
				(Italy)

96	Hellenic National Meteorological Service (Greece)

215	MeteoSwiss (Switzerland)

220	Institute of Meteorology and Water Management
				(Poland)

242	National Meteorological Administration (Romania)

254	EUMETSAT Operation Centre	10	Tromso (Norway)

20	Maspalomas (Spain)

30	Kangerlussuaq (Greenland)

40	Edmonton (Canada)

50	Bedford (Canada)

60	Gander (Canada)

70	Monterey (United States)

80	Wallops Island (United States)

90	Gilmor Creek (United States)

100	Athens (Greece)

120	Ewa Beach, Hawaii

130	Miami, Florida

140	Lannion (France)

150	Svalbard (Norway)

170	St Denis (La Réunion)

180	Moscow

190	Muscat

200	Khabarovsk

210	Novosibirsk

COMMON CODE TABLE C–13:	Data sub-categories of categories defined by entries in BUFR Table A

DATA CATEGORIES	INTERNATIONAL DATA SUB-CATEGORIES

BUFR Edition 4, Octet 11 in Section 1	BUFR Edition 4, Octet 12 (if = 255, it means

other sub-category or undefined)

CREX Edition 2, nnn in Group	CREX Edition 2, mmm in Group Annnmmm
Annnmmm of Section 1	of Section 1

Code figure	Name	Code figure	Name (corresponding traditional alphanumeric
				codes are in brackets)

0	Surface data – land	0	Hourly synoptic observations from fixed-land stations

(SYNOP)

1	Intermediate synoptic observations from fixed-land

stations (SYNOP)

2	Main synoptic observations from fixed-land stations

(SYNOP)

3	Hourly synoptic observations from mobile-land stations

(SYNOP MOBIL)

4	Intermediate synoptic observations from mobile-land

stations (SYNOP MOBIL)

5	Main synoptic observations from mobile-land stations

(SYNOP MOBIL)

6	One-hour observations from automated stations

7	n-minute observations from AWS stations

10	Routine aeronautical observations (METAR)

11	Special aeronautical observations (SPECI)

14	Ground-based GPS humidity observations (GPSIWV)

20	Climatological observations (CLIMAT)

30	Sferics locations

40	Hydrologic reports

50	Hourly synoptic observations with supplementary

one-hour data

51	Intermediate synoptic observations with supplementary

one-hour data

52	Main synoptic observations with supplementary one-

hour data

1	Surface data – sea	0	Synoptic observations (SHIP)

6	One-hour observations from automated stations

7	n-minute observations from AWS stations

20	Climatological observations (CLIMAT SHIP)

25	Buoy observation (BUOY)

30	Tide gauge

31	Observed water level time series

2	Vertical soundings (other	1	Upper-wind reports from fixed-land stations (PILOT)

than satellite)	2	Upper-wind reports from ships (PILOT SHIP)

3	Upper-wind reports from mobile land stations (PILOT

MOBIL)

4	Upper-level temperature/humidity/wind reports from

fixed-land stations (TEMP)

5	Upper-level temperature/humidity/wind reports from

ships (TEMP SHIP)

6	Upper-level temperature/humidity/wind report from

mobile land stations (TEMP MOBIL)

DATA CATEGORIES	INTERNATIONAL DATA SUB-CATEGORIES

BUFR Edition 4, Octet 11 in Section 1	BUFR Edition 4, Octet 12 (if = 255, it means

other sub-category or undefined)

CREX Edition 2, nnn in Group Annnmmm	CREX Edition 2, mmm in Group Annnmmm
of Section 1	of Section 1

Code figure	Name	Code figure	Name (corresponding traditional
				alphanumeric codes are in brackets)

2	Vertical soundings (other	7	Upper-level temperature/humidity/wind reports from

than satellite)		dropwinsondes (TEMP DROP)

(continued)	10	Wind profiler reports

11	RASS temperature profiles

20	ASDAR/ACARS profiles (AMDAR)

21	Profiles of atmospheric constituents concentrations

25	Climatological observations from fixed-land stations

(CLIMAT TEMP)

26	Climatological observations from ships (CLIMAT TEMP

SHIP)

3	Vertical soundings	0	Temperature (SATEM)

(satellite)	1	TIROS (TOVS)

2	ATOVS

3	AMSU-A

4	AMSU-B

5	HIRS

6	MHS

7	IASI

20	IR temperature/humidity sounding

30	Hyperspectral temperature/humidity sounding

40	MW temperature/humidity sounding

50	Radio occultation sounding

4	Single level upper-air data  	0	ASDAR/ACARS (AMDAR)

(other than satellite)	1	Manual (AIREP, PIREP)

5	Single level upper-air data	0	Cloud wind data (SATOB)
		(satellite)	1	Cloud properties

6	Radar data	0	Reflectivity data

1	Doppler wind profiles

2	Derived products

3	Ground radar weather (RADOB)

7	Synoptic features	0	Forecast tropical cyclone tracks from EPS

1	Squall line

8	Physical/chemical	0	Surface ozone

constituents	1	Ozone vertical sounding

2	Total ozone

9	Dispersal and transport	0	Trajectories, analysis or forecast

10	Radiological data	1	Observation (RADREP)

2	Forecast (RADOF)

12	Surface data (satellite)	0	ERS-uwa

1	ERS-uwi

2	ERS-ura

DATA CATEGORIES	INTERNATIONAL DATA SUB-CATEGORIES

BUFR Edition 4, Octet 11 in Section 1	BUFR Edition 4, Octet 12 (if = 255, it means

other sub-category or undefined)

CREX Edition 2, nnn in Group Annnmmm	CREX Edition 2, mmm in Group Annnmmm
of Section 1	of Section 1

Code figure	Name	Code figure	Name (corresponding traditional
				alphanumeric codes are in brackets)

12	Surface data (satellite)	3	ERS-uat

(continued)	4	SSM/I radiometer

5	Quikscat

6	Surface temp./radiation (SATOB)

7	ASCAT data

8	Soil moisture

9	Normalized differential vegetation index (NDVI)

10	Normalized radar backscatter

11	Surface emissivity

12	Sea-surface temperature

21	Radiances (satellite	0	Earth radiation budget

measured)	5	Cross-track infrared sounder

6	Advanced technology microwave sounder

7	Visible/infrared imager radiometer suite

22	Radar (satellite) but not 	0	Cloud and precipitation radar

altimeter and scatterometer	1	Synthetic aperture radar

23	Lidar (satellite)	0	Lidar based missions (for wind, for cloud/aerosol,
				for water vapour, for altimetry)

24	Scatterometry (satellite)	0	Wind scatterometry

25	Altimetry (satellite)	0	Radar altimetry

26	Spectrometry (satellite)	0	Cross nadir shortwave spectrometry (for chemistry)

1	Cross nadir IR spectrometry (for chemistry)

2	Limb sounding shortwave spectrometry

3	Limb sounding IR spectrometry

4	Limb sounding sub-millimetre wave spectrometry

30	Calibration dataset	0	Subsetted data

(satellite)	1	Collocated data

2	On-board calibration data

3	Bias monitoring

4	Near-real-time correction

5	Re-analysis correction

31	Oceanographic data	0	Surface observation

1	Surface observation along track (TRACKOB)

2	Spectral wave observation (WAVEOB)

3	Bathythermal observation (BATHY)

4	Sub-surface floats (profile)

5	XBT/XCTD profiles (TESAC)

6	Waves reports

7	Tsunameter data

DATA CATEGORIES	INTERNATIONAL DATA SUB-CATEGORIES

BUFR Edition 4, Octet 11 in Section 1	BUFR Edition 4, Octet 12 (if = 255, it means

other sub-category or undefined)

CREX Edition 2, nnn in Group Annnmmm	CREX Edition 2, mmm in Group Annnmmm
of Section 1	of Section 1

Code figure	Name	Code figure	Name (corresponding traditional
				alphanumeric codes are in brackets)

101	Image data (satellite)	0	Multi-purpose VIS/IR imagery

1	Conical scanning MW imagery
				(intermediate frequencies)

2	Low frequency MW imagery

3	Ocean colour imagery

4	Imagery with special viewing geometry

5	Lightning imagery

6	High-resolution shortwave imagery for land observation

7	SMOS data

COMMON CODE TABLE C–14:	Atmospheric chemical or physical constituent type

Common Code table	Code table 4.230 in GRIB Edition 2

Code figure	Meaning	Chemical formula

0	Ozone	O3

1	Water vapour	H2O

2	Methane	CH4

3	Carbon dioxide	CO2

4	Carbon monoxide	CO

5	Nitrogen dioxide	NO2

6	Nitrous oxide	N2O

7	Formaldehyde	HCHO

8	Sulphur dioxide	SO2

9	Ammonia	NH3

10	Ammonium	NH4+

11	Nitrogen monoxide	NO

12	Atomic oxygen	O

13	Nitrate radical	NO3

14	Hydroperoxyl radical	HO2

15	Dinitrogen pentoxide	N2O5

16	Nitrous acid	HONO

17	Nitric acid	HNO3

18	Peroxynitric acid	HO2NO2

19	Hydrogen peroxide	H2O2

20	Molecular hydrogen	H

21	Atomic nitrogen	N

22	Sulphate	SO42–

23	Radon	Rn

24	Elemental mercury	Hg(0)

25	Divalent mercury	Hg2+

26	Atomic chlorine	Cl

27	Chlorine monoxide	ClO

28	Dichlorine peroxide	Cl2O2

29	Hypochlorous acid	HClO

30	Chlorine nitrate	ClONO2

31	Chlorine dioxide	ClO2

32	Atomic bromine	Br

33	Bromine monoxide	BrO

34	Bromine chloride	BrCl

35	Hydrogen bromide	HBr

36	Hypobromous acid	HBrO

37	Bromine nitrate	BrONO2

38	Oxygen	O2

39-9999	Reserved

10000	Hydroxyl radical	OH

10001	Methyl peroxy radical	CH3O2

10002	Methyl hydroperoxide	CH3O2H

10004	Methanol	CH3OH

10005	Formic acid	CH3OOH

10006	Hydrogen cyanide	HCN

10007	Aceto nitrile	CH3CN

10008	Ethane	C2H6

10009	Ethene (= Ethylene)	C2H4

10010	Ethyne (= Acetylene)	C2H2

10011	Ethanol	C2H5OH

10012	Acetic acid	C2H5OOH

10013	Peroxyacetyl nitrate	CH3C(O)OONO2

10014	Propane	C3H8

10015	Propene	C3H6

Code figure	Meaning	Chemical formula

10016	Butanes	C4H10

10017	Isoprene	C5H10

10018	Alpha pinene	C10H16

10019	Beta pinene	C10H16

10020	Limonene	C10H16

10021	Benzene	C6H6

10022	Toluene	C7H8

10023	Xylene	C8H10

10024–10499	Reserved for other simple organic molecules
		(e.g. higher aldehydes, alcohols, peroxides,…)

10500	Dimethyl sulphide	CH3SCH3 (DMS)

10501–20000	Reserved

20001	Hydrogen chloride

20002	CFC-11

20003	CFC-12

20004	CFC-113

20005	CFC-113a

20006	CFC-114

20007	CFC-115

20008	HCFC-22

20009	HCFC-141b

20010	HCFC-142b

20011	Halon-1202

20012	Halon-1211

20013	Halon-1301

20014	Halon-2402

20015	Methyl chloride (HCC-40)

20016	Carbon tetrachloride (HCC-10)

20017	HCC-140a	CH3CCl3

20018	Methyl bromide (HBC-40B1)

20019	Hexachlorocyclohexane (HCH)

20020	Alpha hexachlorocyclohexane

20021	Hexachlorobiphenyl (PCB-153)

20022–29999	Reserved

30000	Radioactive pollutant (tracer, defined by originating centre)

30001–30009	Reserved

30010	Hydrogen	H-3

30011	Hydrogen organic bounded	H-3o

30012	Hydrogen inorganic	H-3a

30013	Beryllium 7	Be-7

30014	Beryllium 10	Be-10

30015	Carbon 14	C-14

30016	Carbon 14 CO2	C-14CO2

30017	Carbon 14 other gases	C-14og

30018	Nitrogen 13	N-13

30019	Nitrogen 16	N-16

30020	Fluorine 18	F-18

30021	Sodium 22	Na-22

30022	Phosphate 32	P-32

30023	Phosphate 33	P-33

30024	Sulphur 35	S-35

30025	Chlorine 36	Cl-36

30026	Potassium 40	K-40

30027	Argon 41	Ar-41

30028	Calcium 41	Ca-41

30029	Calcium 45	Ca-45

30030	Titanium 44	Ti-44

Code figure	Meaning	Chemical formula

30031	Scandium 46	Sc-46

30032	Vanadium 48	V-48

30033	Vanadium 49	V-49

30034	Chrome 51	Cr-51

30035	Manganese 52	Mn-52

30036	Manganese 54	Mn-54

30037	Iron 55	Fe-55

30038	Iron 59	Fe-59

30039	Cobalt 56	Co-56

30040	Cobalt 57	Co-57

30041	Cobalt 58	Co-58

30042	Cobalt 60	Co-60

30043	Nickel 59	Ni-59

30044	Nickel 63	Ni-63

30045	Zinc 65	Zn-65

30046	Gallium 67	Ga-67

30047	Gallium 68	Ga-68

30048	Germanium 68	Ge-68

30049	Germanium 69	Ge-69

30050	Arsenic 73	As-73

30051	Selenium 75	Se-75

30052	Selenium 79	Se-79

30053	Rubidium 81	Rb-81

30054	Rubidium 83	Rb-83

30055	Rubidium 84	Rb-84

30056	Rubidium 86	Rb-86

30057	Rubidium 87	Rb-87

30058	Rubidium 88	Rb-88

30059	Krypton 85	Kr-85

30060	Krypton 85 metastable	Kr-85m

30061	Krypton 87	Kr-87

30062	Krypton 88	Kr-88

30063	Krypton 89	Kr-89

30064	Strontium 85	Sr-85

30065	Strontium 89	Sr-89

30066	Strontium 89/90	Sr-8990

30067	Strontium 90	Sr-90

30068	Strontium 91	Sr-91

30069	Strontium 92	Sr-92

30070	Yttrium 87	Y-87

30071	Yttrium 88	Y-88

30072	Yttrium 90	Y-90

30073	Yttrium 91	Y-91

30074	Yttrium 91 metastable	Y-91m

30075	Yttrium 92	Y-92

30076	Yttrium 93	Y-93

30077	Zirconium 89	Zr-89

30078	Zirconium 93	Zr-93

30079	Zirconium 95	Zr-95

30080	Zirconium 97	Zr-97

30081	Niobium 93 metastable	Nb-93m

30082	Niobium 94	Nb-94

30083	Niobium 95	Nb-95

30084	Niobium 95 metastable	Nb-95m

30085	Niobium 97	Nb-97

30086	Niobium 97 metastable	Nb-97m

30087	Molybdenum 93	Mo-93

30088	Molybdenum 99	Mo-99

Code figure	Meaning	Chemical formula

30089	Technetium 95 metastable	Tc-95m

30090	Technetium 96	Tc-96

30091	Technetium 99	Tc-99

30092	Technetium 99 metastable	Tc-99m

30093	Rhodium 99	Rh-99

30094	Rhodium 101	Rh-101

30095	Rhodium 102 metastable	Rh-102m

30096	Rhodium 103 metastable	Rh-103m

30097	Rhodium 105	Rh-105

30098	Rhodium 106	Rh-106

30099	Palladium 100	Pd-100

30100	Palladium 103	Pd-103

30101	Palladium 107	Pd-107

30102	Ruthenium 103	Ru-103

30103	Ruthenium 105	Ru-105

30104	Ruthenium 106	Ru-106

30105	Silver 108 metastable	Ag-108m

30106	Silver 110 metastable	Ag-110m

30107	Cadmium 109	Cd-109

30108	Cadmium 113 metastable	Cd-113m

30109	Cadmium 115 metastable	Cd-115m

30110	Indium 114 metastable	In-114m

30111	Tin 113	Sn-113

30112	Tin 119 metastable	Sn-119m

30113	Tin 121 metastable	Sn-121m

30114	Tin 122	Sn-122

30115	Tin 123	Sn-123

30116	Tin 126	Sn-126

30117	Antimony 124	Sb-124

30118	Antimony 125	Sb-125

30119	Antimony 126	Sb-126

30120	Antimony 127	Sb-127

30121	Antimony 129	Sb-129

30122	Tellurium 123 metastable	Te-123m

30123	Tellurium 125 metastable	Te-125m

30124	Tellurium 127	Te-127

30125	Tellurium 127 metastable	Te-127m

30126	Tellurium 129	Te-129

30127	Tellurium 129 metastable	Te-129m

30128	Tellurium 131 metastable	Te-131m

30129	Tellurium 132	Te-132

30130	Iodine 123	I-123

30131	Iodine 124	I-124

30132	Iodine 125	I-125

30133	Iodine 126	I-126

30134	Iodine 129	I-129

30135	Iodine 129 elementary gaseous	I-129g

30136	Iodine 129 organic bounded	I-129o

30137	Iodine 131	I-131

30138	Iodine 131 elementary gaseous	I-131g

30139	Iodine 131 organic bounded	I-131o

30140	Iodine 131 gaseous elementary and organic bounded	I-131go

30141	Iodine 131 aerosol	I-131a

30142	Iodine 132	I-132

30143	Iodine 132 elementary gaseous	I-132g

30144	Iodine 132 organic bounded	I-132o

30145	Iodine 132 gaseous elementary and organic bounded	I-132go

Code figure	Meaning	Chemical formula

30146	Iodine 132 aerosol	I-132a

30147	Iodine 133	I-133

30148	Iodine 133 elementary gaseous	I-133g

30149	Iodine 133 organic bounded	I-133o

30150	Iodine 133 gaseous elementary and organic bounded	I-133go

30151	Iodine 133 aerosol	I-133a

30152	Iodine 134	I-134

30153	Iodine 134 elementary gaseous	I-134g

30154	Iodine 134 organic bounded	I-134o

30155	Iodine 135	I-135

30156	Iodine 135 elementary gaseous	I-135g

30157	Iodine 135 organic bounded	I-135o

30158	Iodine 135 gaseous elementary and organic bounded	I-135go

30159	Iodine 135 aerosol	I-135a

30160	Xenon 131 metastable	Xe-131m

30161	Xenon 133	Xe-133

30162	Xenon 133 metastable	Xe-133m

30163	Xenon 135	Xe-135

30164	Xenon 135 metastable	Xe-135m

30165	Xenon 137	Xe-137

30166	Xenon 138	Xe-138

30167	Xenon sum of all Xenon isotopes	Xe-sum

30168	Caesium 131	Cs-131

30169	Caesium 134	Cs-134

30170	Caesium 135	Cs-135

30171	Caesium 136	Cs-136

30172	Caesium 137	Cs-137

30173	Barium 133	Ba-133

30174	Barium 137 metastable	Ba-137m

30175	Barium 140	Ba-140

30176	Cerium 139	Ce-139

30177	Cerium 141	Ce-141

30178	Cerium 143	Ce-143

30179	Cerium 144	Ce-144

30180	Lanthanum 140	La-140

30181	Lanthanum 141	La-141

30182	Praseodymium 143	Pr-143

30183	Praseodymium 144	Pr-144

30184	Praseodymium 144 metastable	Pr-144m

30185	Samarium 145	Sm-145

30186	Samarium 147	Sm-147

30187	Samarium 151	Sm-151

30188	Neodymium 147	Nd-147

30189	Promethium 146	Pm-146

30190	Promethium 147	Pm-147

30191	Promethium 151	Pm-151

30192	Europium 152	Eu-152

30193	Europium 154	Eu-154

30194	Europium 155	Eu-155

30195	Gadolinium 153	Gd-153

30196	Terbium 160	Tb-160

30197	Holmium 166 metastable	Ho-166m

30198	Thulium 170	Tm-170

30199	Ytterbium 169	Yb-169

30200	Hafnium 175	Hf-175

30201	Hafnium 181	Hf-181

30202	Tantalum 179	Ta-179

Code figure	Meaning	Chemical formula

30203	Tantalum 182	Ta-182

30204	Rhenium 184	Re-184

30205	Iridium 192	Ir-192

30206	Mercury 203	Hg-203

30207	Thallium 204	Tl-204

30208	Thallium 207	Tl-207

30209	Thallium 208	Tl-208

30210	Thallium 209	Tl-209

30211	Bismuth 205	Bi-205

30212	Bismuth 207	Bi-207

30213	Bismuth 210	Bi-210

30214	Bismuth 211	Bi-211

30215	Bismuth 212	Bi-212

30216	Bismuth 213	Bi-213

30217	Bismuth 214	Bi-214

30218	Polonium 208	Po-208

30219	Polonium 210	Po-210

30220	Polonium 212	Po-212

30221	Polonium 213	Po-213

30222	Polonium 214	Po-214

30223	Polonium 215	Po-215

30224	Polonium 216	Po-216

30225	Polonium 218	Po-218

30226	Lead 209	Pb-209

30227	Lead 210	Pb-210

30228	Lead 211	Pb-211

30229	Lead 212	Pb-212

30230	Lead 214	Pb-214

30231	Astatine 217	At-217

30232	Radon 219	Rn-219

30233	Radon 220	Rn-220

30234	Radon 222 	Rn-222

30235	Francium 221	Fr-221

30236	Francium 223	Fr-223

30237	Radium 223	Ra-223

30238	Radium 224	Ra-224

30239	Radium 225	Ra-225

30240	Radium 226	Ra-226

30241	Radium 228	Ra-228

30242	Actinium 225	Ac-225

30243	Actinium 227	Ac-227

30244	Actinium 228	Ac-228

30245	Thorium 227	Th-227

30246	Thorium 228	Th-228

30247	Thorium 229	Th-229

30248	Thorium 230	Th-230

30249	Thorium 231	Th-231

30250	Thorium 232	Th-232

30251	Thorium 234	Th-234

30252	Protactinium 231	Pa-231

30253	Protactinium 233	Pa-233

30254	Protactinium 234 metastable	Pa-234m

30255	Uranium 232	U-232

30256	Uranium 233	U-233

30257	Uranium 234	U-234

30258	Uranium 235	U-235

30259	Uranium 236	U-236

Code figure	Meaning	Chemical formula

30260	Uranium 237	U-237

30261	Uranium 238	U-238

30262	Plutonium 236	Pu-236

30263	Plutonium 238	Pu-238

30264	Plutonium 239	Pu-239

30265	Plutonium 240	Pu-240

30266	Plutonium 241	Pu-241

30267	Plutonium 242	Pu-242

30268	Plutonium 244	Pu-244

30269	Neptunium 237	Np-237

30270	Neptunium 238	Np-238

30271	Neptunium 239	Np-239

30272	Americium 241	Am-241

30273	Americium 242	Am-242

30274	Americium 242 metastable	Am-242m

30275	Americium 243	Am-243

30276	Curium 242	Cm-242

30277	Curium 243	Cm-243

30278	Curium 244	Cm-244

30279	Curium 245	Cm-245

30280	Curium 246	Cm-246

30281	Curium 247	Cm-247

30282	Curium 248	Cm-248

30283	Curium 243/244	Cm-243244

30284	Plutonium 238/Americium 241	Pu-238Am-241

30285	Plutonium 239/240	Pu-239240

30286	Berkelium 249	Bk-249

30287	Californium 249	Cf-249

30288	Californium 250	Cf-250

30289	Californium 252	Cf-252

30290	Sum aerosol particulates	SumAer

30291	Sum Iodine	SumIod

30292	Sum noble gas	SumNG

30293	Activation gas	ActGas

30294	Cs-137 Equivalent	EquCs137

30295–59999	Reserved

60000	HOx radical (OH+HO2)

60001	Total inorganic and organic 	RO2
		peroxy radicals (HO2 + RO2)

60002	Passive Ozone

60003	NOx expressed as nitrogen	NOx

60004	All nitrogen oxides (NOy) expressed as nitrogen	NOy

60005	Total inorganic chlorine	Clx

60006	Total inorganic bromine	Brx

60007	Total inorganic chlorine except HCl, ClONO2: ClOx

60008	Total inorganic bromine except HBr, BrONO2: BrOx

60009	Lumped alkanes

60010	Lumped alkenes

60011	Lumped aromatic compounds

60012	Lumped terpenes

60013	Non-methane volatile organic compounds	NMVOC

expressed as carbon

60014	Anthropogenic non-methane volatile organic 	aNMVOC

compounds expressed as carbon

60015	Biogenic non-methane volatile organic compounds 	bNMVOC

expressed as carbon

60016	Lumped oxygenated hydrocarbons	OVOC

60017	NOx expressed as nitrogen dioxide (NO2)	NOx

60018–61999	Reserved

Code figure	Meaning	Chemical formula

62000	Total aerosol

62001	Dust dry

62002	Water in ambient

62003	Ammonium dry

62004	Nitrate dry

62005	Nitric acid trihydrate

62006	Sulphate dry

62007	Mercury dry

62008	Sea salt dry

62009	Black carbon dry

62010	Particulate organic matter dry

62011	Primary particulate organic matter dry

62012	Secondary particulate organic matter dry

62013	Black carbon hydrophilic dry

62014	Black carbon hydrophobic dry

62015	Particulate organic matter hydrophilic dry

62016	Particulate organic matter hydrophobic dry

62017	Nitrate hydrophilic dry

62018	Nitrate hydrophobic dry

62019	Reserved

62020	Smoke – high absorption

62021	Smoke – low absorption

62022	Aerosol – high absorption

62023	Aerosol – low absorption

62024–65534	Reserved

65535	Missing

____________