BUFR Table D – List of common sequences

F	X	Category of sequences	Identifier

NONE

Category 04 – Meteorological sequences common to satellite observations

TABLE

REFERENCE	TABLE	ELEMENT NAME	ELEMENT	Identifier

F  X    Y	REFERENCE		DESCRIPTION

(BUFR sequence for describing satellites contributing to an observed	2017-2.4.5(CM-I)
			geophysical quantity)

3 04 038	1 08 000	Delayed replication of 8 descriptors

0 31 001	Delayed descriptor replication factor

0 08 021	Time significance	= 28 Start of scan

3 01 011	Year, month, day

3 01 013	Hour, minute, second

0 08 021	Time significance	= 29 End of scan

3 01 011	Year, month, day

3 01 013	Hour, minute, second

0 01 007	Satellite identifier

0 02 019	Satellite instruments

0 08 021	Time significance	Set to missing (cancel)

Category 06 – Meteorological or oceanographic sequences 
common to oceanographic observations

TABLE

REFERENCE	TABLE	ELEMENT NAME	ELEMENT	Identifier

F  X    Y	REFERENCE		DESCRIPTION

(Chlorophyll-A (fluorescence) profile data)	2016-3.2.7(DRMM-IV)

3 06 044	1 09 000	Delayed replication of 9 descriptors

0 31 002	Extended delayed descriptor replication factor	Gives number of depths

0 07 062	Depth below sea/water surface	Set to missing

0 08 080	Qualifier for quality class	Set to missing

0 33 050	Global GTSPP quality flag	Set to missing

0 07 065	Water pressure

0 08 080	Qualifier for quality class	Set to 10 (pressure at a level)

0 33 050	Global GTSPP quality flag

0 41 002	Fluorescence	Chlorophyll-A | In kg l-1 
				(= 109 mg m-3)

0 08 080	Qualifier for quality class	Set to 21 (Chlorophyll-A at a level)

0 33 050	Global GTSPP quality flag

(Dissolved nitrate profile data)	2016-3.2.7(DRMM-IV)

3 06 045	1 09 000	Delayed replication of 9 descriptors

0 31 002	Extended delayed descriptor replication factor	Number of depths

0 07 062	Depth below sea/water surface	Set to missing

0 08 080	Qualifier for quality class 	Set to missing

0 33 050	Global GTSPP quality flag	Set to missing

0 07 065	Water pressure

0 08 080	Qualifier for quality class	Set to 10 (pressure at a 	level)

0 33 050	Global GTSPP quality flag

0 41 003	Dissolved nitrate	In μmol kg-1

0 08 080	Qualifier for quality class	Set to 22 (nitrate at a level)

0 33 050	Global GTSPP quality flag

(pH profile data) 	2016-3.2.7(DRMM-IV)

3 06 046	1 09 000	Delayed replication of 9 descriptors

0 31 002	Extended delayed descriptor replication factor	Number of depths

0 07 062	Depth below sea/water surface	Set to missing

0 08 080	Qualifier for quality class 	Set to missing

0 33 050	Global GTSPP quality flag	Set to missing

0 07 065	Water pressure

0 08 080	Qualifier for quality class	Set to 10 (pressure at a level)

0 33 050	Global GTSPP quality flag

0 13 161	pH scale

0 13 080	pH	Dimensionless

0 08 080	Qualifier for quality class	Set to 23 (pH at a level)

0 33 050	Global GTSPP quality flag

(Backscatter profile data) 	2016-3.2.7(DRMM-IV)

3 06 047	0 02 071	Spectrographic wavelength	In m

1 09 000	Delayed replication of 9 descriptors

0 31 002	Extended delayed descriptor replication factor	Number of depths

0 07 062	Depth below sea/water surface	Set to missing

0 08 080	Qualifier for quality class 	Set to missing

0 33 050	Global GTSPP quality flag	Set to missing

0 07 065	Water pressure

0 08 080	Qualifier for quality class	Set to 10 (pressure at a 	level)

0 33 050	Global GTSPP quality flag

0 41 006	Backscattering	m-1

0 08 080	Qualifier for quality class	Set to 24 (backscatter at a level)

0 33 050	Global GTSPP quality flag

Category 15 – Oceanographic report sequences

TABLE

REFERENCE	TABLE	ELEMENT NAME	ELEMENT	Identifier

F  X    Y	REFERENCE	DESCRIPTION

(Met-ocean observations from unmanned surface vehicles)

3 15 011	3 01 150	WIGOS identifier	2018-2.4.4(CM-III)

0 01 036	Agency in charge of operating the observing

platform

0 01 085	Observing platform manufacturer’s model

0 01 086	Observing platform manufacturer’s serial

0 03 001	Surface station type

2 08 032	Change width of CCITT IA5 field

0 01 079	Unique ID for profile	UUID for report, 32

character hex string

2 08 000	Change width of CCITT IA5 field

3 01 011	Year, month, day

3 01 012	Hour, minute

3 01 021	Latitude/longitude (high accuracy)

0 01 012	Direction of motion of moving observing

0 01 013	Speed of motion of moving platform

1 03 000	Delayed replication of 3 descriptor

0 31 000	Short delayed descriptor replication factor

0 07 031	Height of barometer above mean sea level

3 06 038	Sequence for representation of standard surface

marine meteorological observations from

moored buoys

0 12 161	Skin temperature

1 01 000	Delayed replication of 1 descriptors

0 31 000	Short delayed descriptor replication factor

3 06 034	Surface current

1 01 000	Delayed replication of 1 descriptor

0 31 000	Short delayed descriptor replication factor

3 06 039	Sequence for representation of basic wave

measurements

1 01 000	Delayed replication of 1 descriptors

0 31 000	Short delayed descriptor replication factor

3 06 033	Surface salinity

1 01 000	Delayed replication of 1 descriptor

0 31 000	Short delayed descriptor replication factor

3 06 041	Depth and temperature profile (high

accuracy/precision)

1 01 000	Delayed replication of 1 descriptors

0 31 000	Short delayed descriptor replication factor

3 06 004	Depth, temperature, salinity

1 01 000	Delayed replication of 1 descriptor

0 31 000	Short delayed descriptor replication factor

3 06 005	Sub-surface current measurements

1 05 000	Delayed replication of 5 descriptors

0 31 000	Short delayed descriptor replication factor

0 41 001	pCO2

0 08 043	Atmospheric chemical or physical constituent

0 15 028	Mole fraction of atmospheric

constituent/pollutant in dry air

0 08 043	Atmospheric chemical or physical constituent

0 13 080	Water pH

1 04 000	Delayed replication of 4 descriptors

0 31 000	Short delayed descriptor replication factor

0 41 005	Turbidity

0 41 003	Dissolved nitrates

0 22 188	Dissolved oxygen

0 41 002	Fluorescence

1 01 000	Delayed replication of 1 descriptor

0 31 000	Short delayed descriptor replication factor

3 06 040	Sequence for representation of detailed spectral

wave measurements

TABLE

REFERENCE	TABLE	ELEMENT NAME	ELEMENT	Identifier

F  X    Y	REFERENCE	DESCRIPTION

1 04 000	Delayed replication of 4 descriptors

0 31 000	Short delayed descriptor replication factor

0 08 021	Time significance

0 04 025	Time period or displacement

0 14 017	Instantaneous long-wave radiation

0 14 018	Instantaneous short-wave radiation